STSC

Process Technologies Method

and Tool Report

(Volume II)

March 1994

Bruce Allgood

Anne Clough

Gary Cunha

James VanBuren

Process Technology Evaluation Project

Sam Godfrey

Technical Program Manager

This technical report was prepared by the

Software Technology Support Center

Ogden ALC/TISE

Hill AFB, UT   84056

The ideas and findings in this report should not be construed as an official Air Force position.  It is published in the interest of scientific and technical information exchange.

This document is available through the STSC.  To obtain a copy, please contact the STSC Customer Service Office directly at:  Software Technology Support Center, Attn.: Customer Service, Hill AFB, Utah 84056; 801-777-7703 or DSN 458-7703,  Fax 801-777-8069, e-mail  godfreys@wpo.hill.af.mil.

Preface

This document, Volume II of the Process Technologies Report 1994, provides a detailed review of process products, user critiques of products, and IDEF training information.  For more information on the topics in this report, to order copies of Volume I, or to order future copies of Volume II, please contact the STSC customer service department at (801)777-7703 or DSN 458-7703, fax to  (801)777-8069 or DSN 458-8069, or email to godfreys@wpo.hill.af.mil.

Volume I of the Process Technologies Report 1994, is a synopsis of the progress of the Software Technology Support Center (STSC) in evaluating process technologies.  Areas covered in the report include process assessment, process modeling, and process enactment, where process enactment is defined as the use of a formal process definition to guide and control the software process.  Both process methods and computer aided process engineering tools are included.  The report defines process technologies, identifies tools and software engineering environments that support process technologies, discusses the value of emphasizing process in improving software quality, and examines the effective use of process technologies.  The report also attempts to identify the future directions of process technologies to help in planning long-range strategies.

The targets of Volume II of this report are process technologists that have a need for detailed technology-specific information and already have a good understanding of process concepts, terminology, and trends.  The information is aimed at those who must make the decisions about acquiring advanced process technology and prepare their organizations for its effective use.

Although the material presented in this publication has been reviewed for technical accuracy, no guarantees are made or implied.  In particular, though the authors have attempted to incorporate reviewer revisions and suggestions wherever possible, this does not imply that the reviewers concur with the technical accuracy of the report.  In addition, product specifications are subject to change by the vendor without notice.  Therefore readers should always verify information independently and evaluate it in relationship to their needs.

1   Volume II Information	1

Appendix B -  Process Technology Product Sheets	3

Appendix C -  Process Technology Product Critiques	67

Appendix E - IDEF Training Sheets	85

tc " 1   Volume II Information"§ 1   Volume II Information

Volume II of the Process Technologies Method and Tool Report contains detailed information on process products, user product critiques, and IDEF training information.  This volume was intended for those process technologists that already have an understanding of basic process concepts, terminology, and trends.  We feel that readers with this type of need would most likely be product and service vendors, as well as those in the community tasked with researching, buying, or utilizing process products and services.

With new and improved technologies emerging every day, it is a challenge to keep on top of all aspects within the process domain.  We encourage readers and vendors to inform us of any modifications/discrepancies found in this report by contacting the STSC.

The mission of the Software Technology Support Center (STSC) is to transition technologies and exchange information to help DoD Software Development and Support Activities continuously improve their software quality and life cycle productivity.  For this reason, we urge those readers with questions, concerns, and/or a need for assistance with process efforts to contact the STSC customer service department at (801)777-7703 or DSN 458-7703, fax to  (801)777-8069 or DSN 458-8069, or email to godfreys@wpo.hill.af.mil.

(This page is intentionally left blank)

tc "Appendix B -  Process Technology Product Sheets"§Appendix B -  Process Technology Product Sheets

This appendix contains technology information sheets for most of the process technologies and tools in the technology/tool lists.  These sheets provide detailed information on process technologies and tools.  Users of these reports should be able to make preliminary tool assessments based on the provided information.  Information on pricing, contact, support, process technology areas covered, intended users of the technology or tool, intended application area, primary methodology base, hardware platforms, and general tool capabilities is included.  The information in the reports was obtained either directly from the vendor or from the vendor's literature.  In most cases, the vendor has supplied the information.

There are tools in the tool lists for which there is no associated technology information sheet.  This condition occurs because there was insufficient available information to create the technology information sheet, either because the vendor did not supply information in time for publication or because the tool was added to the tool list too late for the creation of a technology information sheet.

The STSC can be contacted for both unpublished and updated reports that may be available.

AGE/ASA by Verilog, Inc.

Product Information:

Version Number:		2.3

Release Date:			12/31/91

Frequency of Updates:	one/year

Date of First Release: 	1986

Number Sold/Released:	1500

Pricing:

Single User Price: 	$15,000

Site License:     	contact salesperson

Multi-Copy Price: 	contact salesperson

Upgrade Costs:	contact salesperson

GSA Price:		not available

Maintenance Price: 	16% of price

Contact Information:

For vendor: in business since: 	1984

For R&D: date research began:	1984

Address:	3010 LBJ Freeway, Ste 900

Dallas, Texas 75234

Point of Contact: 	Wesley Hair

Phone Number:	214-241-6595

Fax Number:		214-241-6594

E-mail Address:

Electronic Bulletin Board:

Customer Support :	1-800-424-3095

Process Technology Areas/Capabilities:

Definition	Modeling

Intended Application Area:

Technical	Real-time

Intended Users of Technology/Tool:

Software Developers         Software Managers

Primary Methodology Base:

Structured Analysis           Petri Nets(available for ASA)                  IDEF

Configurations:

Platform(s):	Sun, HP, IBM, VAX, DEC

Memory Requirements:

Data Import/Export Information:

Description/Purpose:

ASA is a CASE tool based on the IDEF-0 formalism.  Its graphical interface

provides a user-friendly environment to simulate, model and generate test cases for system or software level analysis

STSC Process Technologies Product Sheet Version 2.0b

AI0 for DOS by Knowledge Based Systems International

Product Information:

Version Number:		2.32

Release Date:			Available Now

Frequency of Updates:	As Needed

Date of First Release: 	January, 1989

Number Sold/Released:	250+

Pricing:

Single User Price:

Professional Version, $1695; 	LAN Version, $4995

Discounts:

Discounts for volume sales

Maintenance Price:

Major Revisions (2.1-3.0)

$100 Pers, $500 Prof

Contact Information:

For vendor: in business since: 	1988

For R&D: date research began:	1987

Address:  Knowledge Based Systems, Inc.

2746 Longmire Drive

College Station, TX 77845

Point of Contact: 	Sally Penick

Phone Number:	(409) 260-5274

Fax Number:    	(409) 260-1965

E-mail Address:

Electronic Bulletin Board:   (409) 260-4606

Customer Support :     William Bosshard

Process Technology Areas/Capabilities:

Definition	Process Examples	Enactment (planned 12/92)

Modeling	Asset Library

Intended Application Area:

Technical	MIS	Real-time

Intended Users of Technology/Tool:

Software Developers              Software Managers             Process Engineers

Primary Methodology Base:

IDEF

Configurations:

Platform(s):	PC/DOS

Memory Requirements:	640K

Data Import/Export Information:	Neutral ASCII Text File containing all model 				information (SGML File)

Description/Purpose:

An interactive PC-based tool for function analysis using hierarchical function

modeling based on the Air Force IDEF0 Function Modeling Method.  Created

for PC's running DOS.

STSC Process Technologies Product Sheet Version 2.0b

AI0 for Windows by Knowledge Based Systems International

Product Information:

Version Number:		2.0

Release Date:			Available Now

Frequency of Updates:	As Needed

Date of First Release: 	May 1993

Number Sold/Released:	not avail.

Pricing:

Single User Price: 	$ 2995

Multi-Copy Price: 	Quoted upon request

Upgrade Costs:	Inc. in maintenance

Maintenance Price:

Contact Information:

For vendor: in business since: 	1988

Address:  Knowledge Based Systems, Inc.

2746 Longmire Drive

College Station, TX 77845

Point of Contact: 	Sally Penick

Phone Number:	(409) 260-5274

Fax Number:    	(409) 260-1965

E-mail Address:

Electronic Bulletin Board:   (409) 260-4606

Customer Support :     William Bosshard

Process Technology Areas/Capabilities:

Definition	Process Examples	Enactment (planned 12/92)

Modeling	Asset Library

Intended Application Area:

Technical	MIS	Real-time

Intended Users of Technology/Tool:

Software Developers              Software Managers             Process Engineers

Primary Methodology Base:

IDEF

Configurations:

Platform(s):	PC/DOS Running Windows

Memory Requirements:

Data Import/Export Information:	Neutral ASCII Text File containing all model 				information (SGML File)

Description/Purpose:

Supports all the features for the DOS tool but has several enhancements

allowed by the Windows environment.  The tool is a PC-based tool for

function analysis using hierarchical function modeling based on the Air

Force IDEF0 Function Modeling Method.  Created for PC's running Windows.

Activity Based Costing is another feature that is included.  Supports cut and paste

of model entities between windows.

STSC Process Technologies Product Sheet Version 2.0b

AI4 by Knowledge Based Systems International

Product Information:

Version Number:		N/A

Release Date:

Frequency of Updates:

Date of First Release:

Number Sold/Released:

Pricing:	N/A

Single User Price:

Site License:

Multi-Copy Price:

Upgrade Costs:

GSA Price:

Maintenance Price:

Contact Information:

For vendor: in business since: 	1988

For R&D: date research began:	1987

Address:  Knowledge Based Systems, Inc.

2746 Longmire Drive

College Station, TX 77845

Point of Contact: 	David N. Rice

Phone Number:	(409) 696-7979

Fax Number:  	(409) 696-7277

E-mail Address:

Electronic Bulletin Board:  (409) 696-7055

Customer Support :    William Bosshard

Process Technology Areas/Capabilities:

Definition	Modeling	Process Examples

Intended Application Area:

Technical	MIS	Real-time

Intended Users of Technology/Tool:

Software Developers               Software Managers

Primary Methodology Base:

Object-oriented

Configurations:

Platform(s):	MS Windows, MAC, Sun XWindows/Motif, [Planned]

Memory Requirements:

Data Import/Export Information:

Description/Purpose:

An interactive Unix-based tool for structured design of object oriented

applications using the Air Force IDEF4 Object-Oriented Design Method.

STSC Process Technologies Product Sheet Version 2.0b

APPL/A by University of Colorado - Arcadia Project

Product Information:    N/A

Version Number:

Release Date:

Frequency of Updates:

Date of First Release:

Number Sold/Released:

Pricing:

None; definition and related publications available upon request.  Public domain.

Contact Information:

For vendor: in business since:

For R&D: date research began:

Address:	Computer Science Dept.

University of Colorado

Boulder, CO 80309-0430

Point of Contact: 	Stanley M. Sutton, Jr.

Phone Number:	303-492-7906

Fax Number:		303-492-2844

E-mail Address:  sutton@cs.colorado.edu

Customer Support:  thru publication/Sutton

Process Technology Areas/Capabilities:

Definition	Modeling            Enactment	Process Examples

Intended Application Area:

Process Programming

Intended Users of Technology/Tool:

Software Developers                  Process Engineers

Primary Methodology Base:

Process Language

Configurations:

Not applicable

Description/Purpose:

APPL/A is a process-programming language defined as an extension to Ada.  It is intended for the writing of programs to serve as process models and process implementations.

STSC Process Technologies Product Sheet Version 2.0b

Apt by University of Colorado - Arcadia Project

Product Information:

Version Number:

Release Date:

Frequency of Updates:

Date of First Release:

Pricing:

None; source can be made available via anonymous FTP.  Public domain.

Support very limited; only major bugs will

be fixed. Currently considered stable and

no enhancements are planned; reasonably robust and in daily use in Arcadia.

Contact Information:

For vendor: in business since:

For R&D: date research began:

Address:	Computer Science Dept	University of Colorado

Boulder, CO 80309-0430

Point of Contact: 	Dennis Heimbigner

Phone Number:	303-492-6643

Fax Number:		303-492-2844

E-mail Address:   dennis@cs.colorado.edu

Electronic Bulletin Board:

Customer Support : 	Very limited

Process Technology Areas/Capabilities:

Enactment

Intended Application Area:

Process Execution

Intended Users of Technology/Tool:

Process Engineers

Primary Methodology Base:

Process Language

Configurations:

Platform(s): It is known to work on Sun 3s and Sparcs.  Others not yet attempted.

Memory Requirements:

Software Dependencies: The original versions of Alex, Ayacc, and the IRIS

development tools (developed in Arcadia); standard Ada

compiler (Verdix or Sunada preferred).

Description/Purpose:

Apt translates programs written in a subset of the APPL/A software-process programming language into an equivalent Ada program that may be compiled and executed.

STSC Process Technologies Product Sheet Version 2.0b

Authormate by Eclectic Solutions

Product Information:

Version Number:		1.2

Release Date:			Jan1994

Frequency of Updates: 	one/year

Date of First Release: 	1987

Number Sold/Released:	Confidential

Pricing:

Single User Price: 	$2495

Site License:		None

Multi-Copy Price: 	$1495-$1995

Upgrade Costs:	None

GSA Price:		None

Maintenance Price: 	1st yr free, $250/yr

Contact Information:

In business since: 	1984

Address:	5580 LaJolla Boulevard

Suite 130

LaJolla, CA 92037-7692

Point of Contact: 	C.A. (Al) Irvine

Phone Number:	(619) 696-7529

Fax Number:		(619) 558-7928

E-mail Address:		None

Electronic Bulletin Board:	None

Customer Support :	(619) 546-0804

Process Technology Areas/Capabilities:

Definition	               Modeling	Metrics Support

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers		Software Managers	Process Engineers

Primary Methodology Base:

IDEF 		SADT

Object-oriented		Structured Analysis

Configurations:

Platform(s):			DOS		OS/2		VAX/VMS

Memory Requirements:	530K Base	  ---		      ---

Data Import/Export Information: 	ASCII Text (all platforms)

Description/Purpose:

Support SADT (Activity and Data Modeling) and IDEF0 (Functional Modeling), Model Maintenance and Reuse, Model Distribution, and Modeling Team (Groupware Support.)

STSC Process Technologies Product Sheet Version 2.0b

AutoSADT by Triune Software Inc.

Product Information:

Version Number:		1.7.3

Release Date:			March '93

Frequency of Updates:	every 6-9 mos.

Date of First Release: 	Spring '89

Pricing:

Single User Price:

$495 (V1.7.3)

$995 (V1.8 for Windows; release 8/93)

$995 (V1.8 for MAC; release 8/93)

Site License:			N/A

Multi-Copy Price: 		Call

GSA Price:			N/A

Maintenance Price: 		TBD

Contact Information:

Address:	2900 Presidential Drive Ste 240

Fairborn, Ohio 45324

Point of Contact:

Phone Number:	(513)427-9900

Fax Number:		(513)427-9964

E-mail Address:	N/A

Customer Support : 	(513)427-9900

Process Technology Areas/Capabilities :

Assessment			Definition

Modeling			Process Examples

Intended Application Area:

MIS			Technical

Intended Users of Technology/Tool:

Software Developers		Process Engineers

Primary Methodology Base:

IDEF			Structured Analysis

Configurations:

Platform(s):			Macintosh (1.7.3, 1.8); Windows (1.8)

Memory Requirements: 	2+ MB (1.7.3); 4MB (1.8)

Data Import/Export Information:	Text export of Glossary and Data

Dictionary			Text import/export of Narrative Text

Description/Purpose:

AutoSADT is primarily a definition tool, supporting an ASCII export facility.

STSC Process Technologies Product Sheet Version 2.0b

BPwin by LogicWorks, Inc

Product Information:

Version Number:		1.0

Release Date:  		Aug 1993

Frequency of Updates:	6 months

Date of First Release: 	Aug 1993

Number Sold/Released:	not avail

Pricing:

Single User Price: 	$1995

Site License:     	Call for Info

Multi-Copy Price: 	Call for Info

Upgrade Costs:	Inc in Maintenance

GSA Price:		15 %

Maintenance Price: 	$395/year

Contact Information:

In business since: 	1988

Address:   1060 Route 206

Princeton, NJ   08540

Point of Contact: 	Ronnette Kelly

Phone Number:

(609)252-1177

Fax Number:		(609)252-1175

E-mail Address: 70262,1135@compuserve.com

Electronic Bulletin Board:

Customer Support :	(800)783-7946

Process Technology Areas/Capabilities:

Definition	             Modeling

Intended Application Area:

Technical	             Activity Based Costing

Intended Users of Technology/Tool:

Software Managers	             Process Engineers

Primary Methodology Base:

IDEF

Configurations:

Platform(s):     PC or Compatible, running MS Windows

Memory Requirements:

Data Import/Export Information:     Supports DDE (Dynamic Data Exchange)

Description/Purpose:

BPwin supports the IDEF0 activity modeling notation, and allows the user to

capture all of the information necessary to create the detail required for a complete

IDEF0 kit.  BPwin also supports Activity Based Costing.

STSC Process Technologies Product Sheet Version 2.0b

CAIS-A by SofTech, Inc.

Product Information:

Version Number:		5.5G

Release Date:			Feb. 1992

Frequency of Updates:  	~ Quarterly

Date of First Release: 	June 1990

Pricing:

Maintained under government contract; free

to government agencies

Contact Information:

In business since: 	1969

Date research began:	1985

Address:	SofTech, Inc.

10875 Rancho Bernardo Rd

Suite 100

San Diego, CA 92127

Point of Contact: 	Gary Pritchett

Phone Number:	(619)451-9318

Fax Number:		(619)451-9327

E-mail Address:	GPritchett@nosc.mil

Process Technology Areas/Capabilities:

Definition	Enactment

Modeling	Process-Driven Framework

Intended Application Area:

Technical	Software Engineering Environment

Intended Users of Technology/Tool:

Software Developers	         Software Managers	Process Engineers

Primary Methodology Base:

Process Language		Entity Process Modeling	Object-oriented

Configurations:

Platform(s):	VAX/VMS    SUN-3/UNIX      SUN-4/UNIX

Memory Requirements:	16MB RAM minimum

Data Import/Export Information:

Description/Purpose:

CAIS-A is an implementation of a software engineering environment framework providing platform independence and supporting tool integration.

STSC Process Technologies Product Sheet Version 2.0b

CaseWare/CM by CaseWare, Inc.

Product Information:

Version Number:        	3.0

Release Date:    		7/92

Frequency of Updates:

Date of First Release: 	1986

Number Sold/Released:

Pricing:

CaseWare/CM 3.0 is available on a floating user interface or on a site license basis.  The cost for a standard system is approximately $4,000 per user.

Contact Information:

For vendor: in business since: 	1986

For R&D: date research began:

Address:	108 Pacifica, 2nd Floor

Irvine, CA 92718-3332

Phone Number: 	(714) 453-2200

Point of Contact: 	Lesli Mangeri

Phone Number:	(703) 848-9272

Fax Number:  	(703) 848-4586

Customer Support : 	1/800/4CWI

Process Technology Areas/Capabilities:

Assessment	Enactment	Process Examples

Definition	Metrics Support	Asset Library

Modeling	Problem Tracking

Intended Application Area:

Technical	MIS	Real-time

Intended Users of Technology/Tool:

Software Developers             Software Managers                  Process Engineers

Primary Methodology Base:

Process Language           Rule-based       Object-oriented          Behavior-oriented

Configurations:

Platform(s):	Sun Sparcs, HP9000s, IBM  RS6000

Memory Requirements:

Data Import/Export Information:

Description/Purpose:

CaseWare/CM is a client/server-based configuration management system

targeted to the UNIX market which provides complete control over development

and maintenance activities.  Once a process model is defined using

CaseWare/CM's modeling language called ACCENT, the product will enforce

the process.  CaseWare/CM has an built-in embedded problem tracking

capability to assist in changes and task generation.

STSC Process Technologies Product Sheet Version 2.0b

Design/CPN by Meta Software Corporation

Product Information:

Version Number:		2.0

Release Date:			Oct. 1993

Date of First Release: 	April, 1989

Number Sold/Released:	100+

Pricing:

Single User Price:	$24,000

Site License:	Quoted upon request

Multi-Copy Price:	Quoted upon request

Upgrade Costs:	Inc in maintenance

GSA Price:	$19,200 single license

Maintenance Price: 	$4000/yr (1st yr free)

Contact Information:

In business since:    	Dec. 1985

Date research began:	1986

Address:	125 Cambridge Park Drive

Cambridge, MA 02140

Phone Number: 	(617) 576-6920

Point of Contact: 	Lou Zand

Phone Number:	(617) 876-9725

Fax Number:		(617) 661-2008

E-mail Address:	seltzer@metasoft.com

Cust. Support:cpn-tech-support@metasoft.com

Process Technology Areas/Capabilities:

Definition	Metrics Support        Modeling	Process Examples

Intended Application Area:   Technical Simulation        All

Intended Users of Technology/Tool:

Software Developers                  Process Engineers                 Analysts

Primary Methodology Base:

IDEF	Petri Nets

Configurations:

Platform(s):        Mac, X-Windows (Sun Solaris, HPUX)

Memory Requirements:	Varies, depending on platform.

Data Import/Export Information:   ASCII, User-definable import/export facility

- IDL, spreadsheets, Activity-Based Costing, CASE (Knowledge ADW), Node Tree

Description/Purpose:

Design/CPN is based on the principles of colored Petri nets (CP-nets) which 	embody mathematical and graphical approaches to system behavior.  Using Design/CPN, you can graphically explore the characteristics and limits of design and create high-quality models within a formal framework.  In addition, the tool is tailored for rigorous analyses of behavioral issues such as concurrency, deadlock, or safety that repeatedly surface in complex distributed systems.

Design/CPN consists of an editor for modeling the system and a simulator for executing the model.  The modeling inscriptions are defined in a version of  Standard ML, a functional programming language.  Using standard ML you can also create code segments for sophisticated animation and instrumentation.  Design/IDEF IDEF0 process models can be automatically converted to dynamic models and transferred to Meta's WorkFlow Analyzer for behavioral simulation and analysis.  In  addition, Design/IDEF can act as the cornerstone for BPR, Workflow, CASE and financial analysis through its export/import facilities.

STSC Process Technologies Product Sheet Version 2.0b

Design/IDEF by Meta Software Corporation

Product Information:

Version Number:		3.0

Release Date:			Jan., 1994

Date of First Release: 	Dec. 1987

Number Sold/Released:	1000+

Pricing:

Single User Price:	$3,995

Site License: 		Quoted upon request

Multi-Copy Price: 	Quoted upon request

Upgrade Costs:	Inc. in maintenance

GSA Price:   		$3196 (1 license)

Maintenance Price: 	$600/yr (1st yr incl)

Contact Information:

In business since:    	Dec. 1985

Date research began:	1986

Address:	125 Cambridge Park Drive

Cambridge, MA 02140

Phone Number:    (617) 576-6920

Point of Contact: 	Patty Kostusiak

Phone Number:	(617) 576-6920  x35

Fax Number:		(617) 661-2008

E-mail Address:    patty@metasoft.com

Cust Support : idef-tech support@metasoft.com

Process Technology Areas/Capabilities:

Definition              Metrics Support            Modeling              Process Examples

Intended Application Area:

Business Process Redesign               Technical            Workflow              MIS

Intended Users of Technology/Tool:

Software Developers                      Managers       Process Engineers and Analysts

Primary Methodology Base:

IDEF0 Process Modeling                           IDEF1X Data Modeling

Configurations:

Platform(s):      MS-Windows 3.X, Mac, X-Windows (Sun Solaris, HPUX, AIX)

Memory Requirements:    4 MB or greater, depending on platform

Data Import/Export Information:    AML, SML, ASCII, ANSI X3.138-1988 IRDS

IDL, spreadsheets, Activity-Based Costing, Node Tree, CASE (Knowledge ADW)

Description/Purpose:

Design/IDEF supports IDEF0, IDEF1, IDEF1X and E-R modeling.  It automates many IDEF drawing tasks: arrow creation, box creation and numbering, labeling, and decomposing; supports top down IDEF0 diagramming; supplies a data dictionary, which stores information about objects you create; includes reporting and maintenance features as well as consistency checking.  Along with IDEF0 functional models, you can create supplementary information modeling diagrams using Design/IDEF's built-in IDEF1, IDEF1X and entity relationship graphics.  Design/IDEF0 models can be used as a front-end to the Design/CPN simulator.  The CPN menu allows you to add inscriptions to your IDEF0 model that describe the behavior of the model for this simulation capability.  Design/IDEF IDEF0 process models can be automatically converted to dynamic models and transferred to Meta's WorkFlow Analyzer for behavioral simulation and analysis.  In  addition, Design/IDEF can act as the cornerstone for BPR, Workflow, CASE and financial analysis through its export/import facilities.

STSC Process Technologies Product Sheet Version 2.0b

Professional DYNAMO Plus (Edu. Only) by Pugh Roberts

Product Information:

Version Number: 3.1 (Educational use only)

Release Date:			May 1990

Frequency of Updates:

Date of First Release:

Number Sold/Released:	100 sold

Pricing:

Single User Price:	$495(full)

$125 (run only)

Site License:		(6 copies) $1000(full)

$750(run only)

Multi-Copy Price:

Upgrade Costs:   	Currently free

Maintenance Price: 	Included

Contact Information:

For vendor: in business since:      1966

Address:	41 William Linskey Way

Cambridge, MA  02142

Point of Contact: 	Nola Shepherd

Phone Number:	(617)864-8880

Fax Number:		617)864-8884

E-mail Addr:    70662.3552@compuserve.com

Electronic Bulletin Board:	none

Customer Support : 		Ted Sjoblom

Process Technology Areas/Capabilities:

Definition	          Modeling

Intended Application Area:   Educational use only

Intended Users of Technology/Tool:  Educational use only

Primary Methodology Base:

Process Language                       System Dynamics

Configurations:

Platform(s):IBM PC+ PC Compatibles

Memory Requirements:256K RAM

HW Requirements: Graphics card & monochrome monitor, 1 floppy drive, MS DOS 	 2.0 or higher

Data Import/Export Information:Export: Lotus 123, Excel, or ASCII output.

Import: Cnvrts spreadsheet data to DYNAMO models.

Description/Purpose:

Key Capabilities: 1)Develop models of complex systems 2) Validate Models

3)Examine Dynamics 4) Test alternate policies and assumptions

5) Report and evaluate results.

Some Features:  User defined reports, Macro capability, On-line help.   Also,

gaming options allows trainees to interactively make decisions and see results

of their choices.  Can be set up as training tool.

STSC Process Technologies Product Sheet Version 2.0b

DYNAMO for Windows by Pugh Roberts

Product Information:

Version Number:	3.11  (3.B)

Release Date:		November 1991

Frequency of Updates:

Date of First Release:

Number Sold/Released:	40 sold

Pricing:

Single User Price: 	$3000 (full usage)

Site License:

Multi-Copy Price:  1-6 $3000 (full usage)

(7+ ) $1800 (full usage)

Upgrade Costs:   Currently free

GSA Price:

Maintenance Price: Included

Contact Information:

For vendor: in business since: ~1966

For R&D: date research began:

Address:	41 William Linskey Way

Cambridge, MA  02142

Point of Contact: 	Nola Shepherd

Phone Number:	(617)864-8880

Fax Number:  	(617)864-8884

E-mail Addr:     70662.3552@compuserve.com

Electronic Bulletin Board:	none

Customer Support : 		Ted Sjoblom

Process Technology Areas/Capabilities:

Definition     	Modeling

Intended Application Area:

Technical	Government	Utilities

Environmental 	Medical	MIS

Real-time	Business/Finance/Economics	Manufacturing

Intended Users of Technology/Tool:     Process Engineers	Managers

Primary Methodology Base: Process Language	    System Dynamics

Configurations:

Platform(s):    IBM PC+ PC Compatibles (Mouse needed)

Memory Requirements:   2MB RAM  5 MB Hard Disk space

HW Requirements: Graphics card & Monochrome monitor, 1 floppy, MS DOS 2.0 	or higher.  MS Windows 3.0 or higher.  Can use OS/2 for larger 	models exceeding DOS' 640KB limit.

Data Import/Export Information:Export: Lotus 123, Excel, or ASCII output

Import: Cnvrts sprdsht data to DYNAMO models.

Description/Purpose:

DYNAMO for Windows contains functionality of Prof DYNAMO Plus but brings several powerful new capabilities to DYNAMO Modelers.  The primary advantages of the windows modules includes the following:

• Built-In interface capability

• Expanded capacity

• Dramatically improved output capabilities

• Multiple Report files can appear on screen simultaneously

STSC Process Technologies Product Sheet Version 2.0b

Professional DYNAMO Plus by Pugh Roberts

Product Information:

Version Number:		3.6

Release Date:			May 1990

Frequency of Updates:

Date of First Release: 	May 1990

Number Sold/Released:	40 sold

Pricing:

Single User Price: 	$2000(full)

$500(run only)

Multi-Copy Price:	(1-6)$2000(full)

$500(run only)

(7+)  $1200(full)    			$500 (run only)

Upgrade Costs:	Currently free

Maintenance Price: 	Include

Contact Information:

For vendor: in business since:    1966

For R&D: date research began:

Address:	41 William Linskey Way

Cambridge, MA  02142

Point of Contact: 	Nola Shepherd

Phone Number:	(617)864-8880

Fax Number:		(617)864-8884

E-mail Addr:   70662.3552@compuserve.com

Electronic Bulletin Board:	none

Customer Support : 		Ted Sjoblom

Process Technology Areas/Capabilities:   Definition	Modeling

Intended Application Area:

Technical	Government	Utilities

Environmental	Medical	Business/Finance

MIS	Manufacturing 	Real-time

Intended Users of Technology/Tool:

Process Engineers                   Managers

Primary Methodology Base:

Process Language                    System Dynamics

Configurations:

Platform(s):IBM PC + PC Compatibles

Memory Requirements: 256K RAM

HW Requirements: Graphics card & Monochrome monitor, 1 floppy drive, MS DOS 	2.0 or higher.  Can use OS/2 for larger models exceeding DOS' 	640KB limit.

Data Import/Export Information:Export:Lotus 123, Excel, or ASCII output

Import:Cnvrts spreadsheet data to DYNAMO models.

Description/Purpose:

Key Capabilities: 1)Develop models of complex systems 2) Validate

Models 3)Examine Dynamics 4) Test alternate policies and assumptions

5) Report and evaluate results.

Some Features:  User defined reports, Macro capability, On-line help.

NOTE:  3.6 has same functionality as 3.1 but is designed for much larger models, and is not aimed at educational use.

STSC Process Technologies Product Sheet Version 2.0b

EAST by SFGL

Product Information:

Version Number:		1.2

Release Date:			Oct. 1992

Frequency of Updates:	1/year

Date of First Release: 	Sept.. 1991

Number Sold/Released:	25

Pricing:

Single User Price: 	$8000 (for 10 users)

Site License:		$155,000

Multi-Copy Price:

Upgrade Costs:	Inc in maintenance

Maintenance Price: 	15% fees

Contact Information:

In business since: 		1991

R&Dresearch began:		1987

Address:	SFGL

14, rue de la Ferme

92100 Boulogne - France

Point of Contact: 	Mark Lichtenhein

Phone Number:	+33 1 46 94 77 77

Fax Number:	+33 1 46 94 77 66

E-mail Addr:    Mark.Lichtenhein@sfgl.sfgl.fr

Customer Support : 	Margo Paine SPIESS

Process Technology Areas/Capabilities:

Assessment	Enactment                     Modeling	      Asset Library

Definition	Process Examples         Process-Driven SEE

Intended Application Area:

Technical		Real-time

Intended Users of Technology/Tool:

Software Developers        	Software Managers

Process Engineers             	Configuration Managers

Primary Methodology Base:      Methodology Independent

Configurations:

Platform(s):   SUN Sparc, IBM RS6000, (HP9000 (700&800) and DEC to follow)

Memory Requirements:	48MB RAM, 140 MB Disk

Data Import/Export Information:  Any object, or group of objects (containing data

manipulated by a set of tools) can be exported from the

environment and used as UNIX files; connections to project

management tools is planned for next versions (end 1993);

import/export facilities for generic forms for documents are

supported through the SGML format.

Description/Purpose:

EAST (the Environment of Advanced Software Technology) is the first

commercially available SEE based on PCTE, covering the whole life-cycle.

EAST integrates software development tools together with the management

tools for software development in a complete environment for building today's

complex software systems.  Existing software life-cycle tools can be added to the environment easily using EAST's " encapsulation" mechanism.

STSC Process Technologies Product Sheet Version 2.0b

EAST --> U.S. Contact

Product Information:

Version Number:

Release Date:

Frequency of Updates:

Date of First Release:

Number Sold/Released:

Pricing:

Single User Price:

Site License:

Multi-Copy Price:

Maintenance Price:

Contact Information:

In business since: 	1974

Address:	16400 Ventura Blvd #303

Encino, CA  91436

Point of Contact: 	Marketing

Phone Number:	818-995-7671

Fax Number:		818-995-4267

E-mail Address:    info@markv.com

Customer Support :	support@markv.com

Process Technology Areas/Capabilities:

Assessment	Definition	Process Examples

Modeling	Enactment	Metrics Support

Asset Library	Process-Supported SEE	Process-Driven SEE

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers                 Software Managers                Process Engineers

Primary Methodology Base:

Entity Process Modeling

Configurations:

Platform(s):     UNIX

Memory Requirements:   32 MB

Data Import/Export Information:  PCTE Store

Description/Purpose:

SEE fuller description on previous page.

STSC Process Technologies Product Sheet Version 2.0b

EASYCASE Professional by Evergreen CASE Tools

Product Information:

Version Number:		4.1

Release Date:			May 1994

Frequency of Updates:	6-18 months

Date of First Release: 	1989

Number Sold/Released:	10,000+

Pricing:

Single User Price: 	Please Call

Site License:		Please Call

Multi-Copy Price: 	Please Call

Upgrade Costs:	Varies

GSA Price:		N/A

Maintenance Price: 	N/A

Contact Information:

In business since: 	1989

Date research began:	1989

Address:	8522 154th Avenue, NE

Redmond, WA 98052

Point of Contact: 	Brian Hansford

Phone Number:	(206) 881-5149

Fax Number:		(206) 883-7676

Customer Support : 	same as above

Process Technology Areas/Capabilities:

Definition	Modeling	Process Examples

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers    	      Software Managers     	Process Engineers

Primary Methodology Base:

Structured Analysis     	Entity Process Modeling	Shlaer-Mellor

Rule-based		IDEF1X	Yourdon/DeMarco

Gane & Sarson		Ward-Mellor	Bachman

Chen		MERISE	SSADM

Yourdon/Constantine		Martin Information Engineering

Configurations:

Platform(s):				DOS/Windows/Networks

Memory Requirements:		4MB RAM, 4 MB disk space. (9 MB if you

want to use it with the samples)

Data Import/Export Information:	dBASE III+, ASCII, SDF

Description/Purpose:

EasyCASE Professional is aimed at the process modeling community, and supports several methodologies.  This tool supports graphical editing with an underlying data dictionary and report generation.

STSC Process Technologies Product Sheet Version 2.0b

EASYCASE System Designer by Evergreen CASE Tools

Product Information:

Version Number:		4.1

Release Date:			May 1994

Frequency of Updates:	6-18 months

Date of First Release: 	1989

Number Sold/Released:	10,000+

Pricing:

Single User Price: 	Please Call

Site License:		Please Call

Multi-Copy Price: 	Please Call

Upgrade Costs:	Varies

GSA Price:		N/A

Maintenance Price: 	N/A

Contact Information:

In business since: 	1989

Date research began:	1989

Address:	8522 154th Avenue, NE

Redmond, WA 98052

Point of Contact: 	Brian Hansford

Phone Number:	(206) 881-5149

Fax Number:		(206) 883-7676

Customer Support : 	same as above

Process Technology Areas/Capabilities:

Definition	Modeling	Process Examples

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers    	      Software Managers     	Process Engineers

Primary Methodology Base:

Structured Analysis     	Entity Process Modeling	Shlaer-Mellor

Rule-based		IDEF1X	Yourdon/DeMarco

Gane & Sarson		Ward-Mellor	Bachman

Chen		MERISE	SSADM

Yourdon/Constantine		Martin Information Engineering

Configurations:

Platform(s):				Windows/Networks

Memory Requirements:		4MB RAM, 5 MB disk space. (12 MB if you

want to use it with the samples)

Data Import/Export Information:   	dBASE III+, ASCII, SDF

Description/Purpose:

EasyCASE System Designer is primarily used for data modeling and data base design,  and supports several methodologies.  It will extract a schema from your E-R diagrams, read xbase tables, and allow you to view your entity attributes right on your charts.

STSC Process Technologies Product Sheet Version 2.0b

ERwin/SQL, ERwin/ERX, ERwin/DBF by LogicWorks, Inc.

Product Information:

Version Number:	2.2, 1.2A, 1.1

Release Date:		3/91,8/91 and 5/91

Frequency of Updates: 6 months

Date of First Release:  1/90, 5/91 and 5/92

Pricing:

Single User Price:  	$1795, $2795, $795

Maintenance Price:  	$325, $495, $175

Site License:     	Call for Info

Multi-Copy Price: 	Call for Info

Upgrade Costs:	Inc in Maintenance

GSA Price:		15 %

Contact Information:

In business since: 	1988

Address:   1060 Route 206

Princeton, NJ 08540

Point of Contact: 	Ronnette Kelly

Phone Number:	(609) 2521177

Fax Number:		(609) 252-1175

E-mail Address: 70262,1135@compuserve.com

Process Technology Areas/Capabilities:

Modeling

Intended Application Area:

Technical	         Real-time	MIS

Intended Users of Technology/Tool:

Software Developers

Primary Methodology Base:

IDEF1X

Configurations:

Platform(s):	SQL:IBM, ERX: IBM/Macintosh and DBF: IBM

Disk Size IBM:   3.5" and 5.25" disk included, Macintosh: 3.5" disk included

System Requirements:    IBM: Microsoft Windows 3.x; Macintosh: System 7

Description/Purpose:

Logic Works ERwin tools are Entity-Relationship (ER) database design tools that

help the user quickly design and build databases using an intuitive,

graphical approach.  Tools support logical data modeling that allows a

modeler to create graphical models of the data in a system and hence can be

used with IDEF0 functional modeling tools to support data modeling where

that is appropriate.

STSC Process Technologies Product Sheet Version 2.0b

Excelerator Series by Intersolv

Product Information:

Version Number:		1.2

Release Date:			4/30/93

Frequency of Updates:	Semi-annual

Date of First Release: 	3/31/92

Number Sold/Released:

Pricing:

Contact vendor for price information.

Contact Information:

In business since: 	1984

Date research began:	1982

Address:	One Main Street

Cambridge, MA 02142

Phone Number: 	(800) 777-8858

Point of Contact: 	Patti Darsigny

Phone Number:	(617) 252-4508

Fax Number:		(617) 494-8591

Customer Support : 	(800) 443-1601

Process Technology Areas/Capabilities:

Assessment	Definition           Modeling	Enactment

Intended Application Area:

MIS

Intended Users of Technology/Tool:

Software Developers

Primary Methodology Base:

Structured Analysis    	       Object-oriented	Methodology Independent

Configurations:

Platform(s):	OS/2, Windows NT (not yet available)

Memory Requirements:	32 mg

Data Import/Export Information:	CDIF, ESF

Description/Purpose:

Excelerator for Planning, Analysis, and Design provides a set of tools that

helps plan, specify, and design MIS applications.  The toolset allows an

organization to set up an environment to support its software development

process.  The modeling tool in this toolset, Excelerator II, supports enterprise

data modeling, enterprise process modeling, IMS database modeling, relational

database modeling, physical data design and program design.  Excelerator II is

still a very new product and its user base is primarily in small pilot projects;

evolving into more widespread use.

STSC Process Technologies Product Sheet Version 2.0b

firstCASE Cross Life Cycle by AGS Management Systems

Product Information:

Version Number:		3.0

Release Date:  		1/93

Frequency of Updates:	Annual

Date of First Release: 	1/90

Pricing:

firstCASE Methodology:	$19,500

firstCASE Automation

Single User System: 	$11,500

Multi-Copy Price: 	Varies

GSA Price:		7% discount

Maintenance Price:   	15% of price

Contact Information:

In business since: 	1969

Address:	880 First Avenue

King of Prussia, PA

19406

Point of Contact: 	Russell Giangiulio

Phone Number:	215-265-1550

Fax Number:		215-265-1230

Customer Support : 	215-265-6305

Process Technology Areas/Capabilities:

Assessment	Enactment	Process-Driven Framework

Modeling	Definition	Process Examples

Metrics Support

Intended Application Area:

All	Technical                   MIS	Real-time

Intended Users of Technology/Tool:

Software Developers          Software Managers         Process Engineers

Primary Methodology Base:

Structured Analysis              	Entity Process Modeling

Behavior-oriented               	Methodology Independent

Configurations:

Platform(s):		Windows 3.1

Memory Requirements:	6-8 MB Extended memory - 60MB hard disk

Data Import/Export Information:	System developed on Gupta SQL Database

Description/Purpose:

firstCASE Cross Life Cycle is an automated methodology that covers Enterprise Architecture  Planning, Information Engineering, and a structured approach for systems development that combines process management, estimating, and project management into one integrated system.

STSC Process Technologies Product Sheet Version 2.0b

HP Softbench by Hewlett Packard

Product Information:

Version Number:		8.00

Release Date:			10/92

Frequency of Updates:	9-12 months

Date of First Release:  	12/89

Number Sold/Released:	20,000

Pricing:

Single User Price: 	$2300

Site License:

Multi-Copy Price:

GSA Price:		Approx 35% discount

Maintenance Price: 	$345/year

Contact Information:

In business since: 	1989

Date research began:	1989

Address:    SW Engineering System Div.

3404 E. Harmony Rd.

Ft. Collins, CO 80525-9599

Phone Number: 	(303) 229-3800

Point of Contact: 	Charlie Doucot

Phone Number:	(617) 221-5191

Process Technology Areas/Capabilities:

Open Framework for support of integrated tools - includes code construction tools

Process-Driven Framework                                     Process-Supported SEE

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers    	                    Software Managers

Primary Methodology Base:

Object-oriented	                     Methodology Independent

Configurations:

Platform(s):  HP, SUN, IBM

Memory Requirements:	Minimum 16 MB

Data Import/Export Information:	Data may be imported and

exported in the S/B environment

Description/Purpose:

Softbench is a software development environment.  Softbench consists of

an integrated set of window-based program development tools, libraries, and

an integration framework.  Softbench also provides class construction, graphical

browsing, diagnostic checking, debugging, building, and other tools which are

integrated into one extensible environment.

STSC Process Technologies Product Sheet Version 2.0b

IDEFine Family by Wizdom Systems

Product Information:

Version Number:		1.51

Release Date:			June 15, 1992

Frequency of Updates:	~ 2/Yr

Date of First Release: 	1987

Number Sold/Released:	120+

Pricing:

Single User Price: 	$8,000

Site License:

Multi-Copy Price:

Upgrade Costs:	Inc in Maintenance

GSA Price:

Maintenance Price:	$1500/yr;  1st Yr Free

Contact Information:

In business since: 	1986

Sate research began:	1986

Address:	1300 Iroquois Ave.

Naperville, IL 60563

Phone Number: 	708-357-3000

Point of Contact: 	Don Sloan

Phone Number:	313-751-6901

Customer Support : 	708-357-3000

Process Technology Areas/Capabilities:

Definition	Process Examples	Metrics Support

Modeling	Asset Library	Process-Driven Framework

Intended Application Area:

Technical	            MIS	Real-time

Intended Users of Technology/Tool:

Software Developers	            Software Managers 	Process Engineers

Primary Methodology Base:

IDEF

Configurations:

Platform(s):	IBM Compatible - DOS 3.1 or later

Memory Requirements:	640K or better

Data Import/Export Information:	IDL, dBASE compatible models

Description/Purpose:

Software designed for IDEF0 and IDEF1X modeling.  Also, activity

based costing analysis, simulation analysis and quality functional deployment

STSC Process Technologies Product Sheet Version 2.0b

IKE by Knowledge Base Engineering, Inc.

Product Information:

Version Number:  		MAC: V1.6;

PC: V1.0

Release Date:        		MAC: 9/1/92;

PC 1/15/93

Frequency of Updates:	every 3 months

Date of First Release: 	MAC: 1/19;

PC: 11/92

Number Sold/Released:	12 copies

Pricing:

Commercial version not available at  this

time. Government purpose only license.

Contact Information:

In business since: 	April, 1986

Date research began:	April, 1986

Address:	71 Rhoads Center Drive

Centerville, OH 45458

Point of Contact: 	Sam or Greg Nusinow

Phone Number:	(513) 434-1716

Fax Number:		(513) 434-4246

Customer Support : 	(513) 434-1716

Process Technology Areas/Capabilities:

Definition	Metrics Support	Process Examples

Modeling	Multi-media capabilities	Process-Driven Framework

Enactment	System Engineering	Process-Driven SEE

Intended Application Area:     All

Intended Users of Technology/Tool:

Software Developers    	      Software Managers     	Process Engineers

Primary Methodology Base:

Entity Process Modeling 	Object-oriented

Behavior-oriented			IDEF

Structured Analysis     		Rule-based

Configurations:

Platform(s):    MAC: II or greater; PC: 386 or greater

Memory Requirements:   MAC: 8MB; PC 4+ MB

Data Import/Export Info: Common multi-media formats and ASCII on each platform

Description/Purpose:

The Integrated Knowledge Environment (IKE) provides advanced authoring

functions for rapidly evolving intelligent multi-media process oriented computer

based training and knowledge capture applications. The IKE architecture contains

easy to use components for dynamically integrating process data, graphics, data

objects, forms, rules, text, video, bit mapped images, and computer-off-the-shelf

software into single or multi-user applications without any computer programming.

IKE is being used by Rockwell on the CALS-IDS program to develop process

models of the engineering and manufacturing operations at the various AF logistics

centers as well as the B1 SPO operations at Tinker Air Force Base.

STSC Process Technologies Product Sheet Version 2.0b

IMDE/COMPASS by TASC

Product Information:

Release Date:	June 1, 1993

Developed by TASC for Armstrong Lab at

Wright-Patterson AFB.

Pricing:

Single User Price: 	TBD

Site License:		TBD

Multi-Copy Price: 	TBD

Upgrade Costs:	TBD

GSA Price:		TBD

Maintenance Price: 	TBD

Contact Information:

In business since: 	1967

Date research began:	3/90

Address:	2555 University Blvd.

Fairborn, OH 45324

Point of Contact: 	Mr. Pat Clark

Phone Number:	(513) 426-1040

Fax Number:		(513) 426-8888

E-mail Address:	pkclark@tasc.com

Process Technology Areas/Capabilities:

Definition	Process Examples

Modeling	Assessment

Object-oriented databases support evolutionary models/model parts and reuse

Intended Application Area:

Technical

Intended Users of Technology/Tool:

Process Engineers	            Anyone doing process modeling and simulation

Primary Methodology Base:      Object-Oriented

Configurations:

Platform(s):	Sun Workstation (SPARC Station)

Memory Requirements:	32 MB main memory; 450 MB disk space

Data Import/Export Information:	Prewritten MODSIM II simulation code

and data files can be imported.  Exports are project and

object structure diagrams and reports, including generated

source code, documentation, raw simulation data, summary

data, and graphs.

Description/Purpose:

The IMDE (commercialized name COMPASS) is a multi-user system for the development and analysis of discrete-event simulations.  It allows the definition, simulation, and data analysis of object-oriented models from a user-defined

domain.  Suitable for the creation of models in the areas of manufacturing, transportation, logistics, communication, etc.  The modeler could easily define a

set of domain objects relevant in the software development process and hence be

used to define and simulate that process.

STSC Process Technologies Product Sheet Version 2.0b

ISEE, TSEE, and RTEE by Westmount USA

Product Information:

Version Number:		3.0

Release Date:			3/93

Frequency of Updates: 	2/yr

Date of First Release: 	1988

Number Sold/Released:	1200

Pricing:

Single User Price: 	$9,975

Site License:		N/A

Multi-Copy Price: 	Discounted

Upgrade Costs:	Inc in maintenance

GSA Price:		N/A

Maintenance Price: 	15% of list

Contact Information:

In business since: 	1986

Date research began:	1987

Address:	Westmount USA

1555 Wilson Blvd. Ste 300

Arlington, VA 22209

Point of Contact: 	Bob Carter

Phone Number:	(703) 875-8799

Fax Number:		(703) 527-5709

E-mail Address:	boca@wmt.nl

Elec. Bulletin Board:	wmt.info@wmt.nl

Customer Support : 	helpdesk@wmt.nl

Process Technology Areas/Capabilities :

Definition	Process Examples	Process-Driven Framework

Modeling	Asset Library	Process-Supported SEE

Intended Application Area:

MIS	          Real-time	Technical

Intended Users of Technology/Tool:

Software Developers    Software Managers     Process Engineers	Programmers

Primary Methodology Base:

Process Language			Structured Analysis

Entity Process Modeling    	Object-oriented

Configurations:

Platform(s):		All major UNIX, VAX/VMS, SLO/UNIX

Memory Requirements:     24 MB (RAM)

Data Import/Export Information:    CDIF Support

Description/Purpose:

ISEE is an integrated Computer Aided Software Engineering (I-CASE)

environment supporting the full software development lifecycle of business

information systems.  Its upper CASE toolset supports process definition

and data modeling using primarily structured analysis and Yourdon

methodology. Westmount provides ISEE for information systems, TSEE

for technical applications and RTEE for real-time.

STSC Process Technologies Product Sheet Version 2.0b

ISPW by Benchmark Technologies

Product Information:

Version Number:		1.4

Release Date:			1992

Frequency of Updates:	18 mo.

Date of First Release  	1988

Number Sold/Released:

Pricing:

$30,000 - $250,00

Maintenance Price:  15% of list price

Contact Information:

In business since: 	1986

Date research began:	1985

Address:	Suite 300, 839 5th Ave. SW

Calgary, Alberta

Canada T2P 3C8

Point of Contact: Mrs. Christina McGill

Phone Number:	(403) 269-7499

Fax Number:		(403) 265-2379

Process Technology Areas/Capabilities:

Process-Supported SEE  	Process-Driven SEE

Change Management	    	Configuration Management

Intended Application Area:

MIS

Intended Users of Technology/Tool:

Software Developers      Software Managers       Quality Assurance	      End Users

Primary Methodology Base:

Methodology Independent

Configurations:

Platform(s):		MVS (IBM Mainframe)

Memory Requirements:

Data Import/Export Information:

Description/Purpose:

ISPW is an automated application management environment spanning many development functions including change and configuration management; it is a framework for tool interaction and a work environment for the management of procedures, methods, technologies, standards, and people under a common control policy.  ISPW can be customized for the organization's own process and standards and provides a comprehensive audit trail.  Support for specialized group and project documentation requirements.  ISPW has integrated a variety of CASE tools to help manage the development process.

STSC Process Technologies Product Sheet Version 2.0b

KI-Shell by UES, Inc.

Product Information:

Version Number:		1.1.3

Release Date:			10/31/93

Frequency of Updates:	6-12 months

Date of First Release: 	Oct. 1990

Number Sold/Released:	Approx. 50

Pricing:

Call for pricing information

Upgrade Costs:	N/A

GSA Price:		N/A

Maintenance Price: 	Call for information

Contact Information:

In business since: 		1973

KI-Shell division since:	1986

Address:        UES, Inc.

5162 Blazer Memorial Pkwy

Dublin, OH 43017

Point of Contact: 	Vijay Shende

Phone Number:	(614) 792-9993

Fax Number:		(614) 792-0998

Customer Support : 	(614) 792-9993

Process Technology Areas/Capabilities:

Definition	Metrics Support	Process-Driven Framework

Modeling	Process Examples	Process-Supported SEE

Enactment	Asset Library	Process-Driven SEE

Intended Application Area:

All	  Technical	MIS

Intended Users of Technology/Tool:

Software Developers  	            Software Managers             Process Engineers

End Users for Enactment

Primary Methodology Base:

Process Language		Structured Analysis    	Petri Nets

IDEF		Rule-based	Object-oriented

Behavior-oriented		Entity Process Modeling	Methodology Independent

Configurations:

Platform(s):	UNIX: IBM RS/6000, HP 9000, SUN SPARC, PC/OS2

Memory Requirements:	16 MB

Data Import/Export Information:	IDEF0, Knowledge Ware, IDEF3 planned,

SQL Tables

Description/Purpose:

Model and implement systems for executing workflow processes between multiple users, provide decision support and tool integration.

STSC Process Technologies Product Sheet Version 2.0b

MethodMaker by Mark V Systems

Product Information:

Version Number:		2.1

Release Date:			August 1993

Frequency of Updates:	twice / year

Date of First Release: 	August 1993

Number Sold/Released:	25

Pricing:

Single User Price: 	$5000

Site License:     	Call for info

Multi-Copy Price: 	Call for info

Maintenance Price: 	15% of cost

Contact Information:

In business since: 	1974

Address:	16400 Ventura Blvd #303

Encino, CA  91436

Point of Contact: 	Marketing

Phone Number:	818-995-7671

Fax Number:		818-995-4267

E-mail Address:	info@markv.com

Customer Support :	support@markv.com

Process Technology Areas/Capabilities:

Definition	Prototyping

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers         Software Managers            Process Engineers

Primary Methodology Base:

Structured Analysis		Petri Nets	Entity Process Modeling

Rule-Based		Object Oriented	Behavior-oriented

IDEF		Methodology Independent

Configurations:

Platform(s):     Most UNIX, VMS and PC Windows

Memory Requirements:     UNIX/VMS - 12 MB              PC - 4 MB

Data Import/Export Information:     Under Development

Description/Purpose:

MethodMaker is a meta method tool generator for developing and extending process and method notations.  MethodMaker supports graphical modeling of a process notation and then automatically generates Process Maker predicate logic rule support.

STSC Process Technologies Product Sheet Version 2.0b

MicroWorld Creator by MicroWorlds, Inc.

Product Information:

Version Number:		2.0

Release Date:			4/92

Frequency of Updates:	1-2 years

Date of First Release: 	1988

Number Sold/Released:	approx. 1000

Pricing:

Single User Price: 	$680.00

Site License:   	Runtime vers. avail.

Multi-Copy Price:

Upgrade Costs:	$85.00

GSA Price:

Maintenance Price: 	No charge

Contact Information:

In business since: 	1988

Date research began:

Address:	47 Third Street, Suite 200

Cambridge, MA 02141

Point of Contact: 	Elizabeth Peterson

Phone Number:	(617) 225-0025

Fax Number:		(617) 225-0028

E-mail Address:	AppleLink: D3640

Electronic Bulletin Board:	--------

Customer Support : 	(617) 225-0025

Process Technology Areas/Capabilities:

Modeling	Process Examples          Process-Driven Framework

Metrics Support	Asset Library

Intended Application Area:

Technical	Real-time

MIS	Software Project Management/Strategy

Intended Users of Technology/Tool:

Software Managers	     Model Developers	Workshop Facilitators

Primary Methodology Base:

System Dynamics             Behavior-oriented

Configurations:

Platform(s):	Macintosh

Memory Requirements:	1 MB.

Data Import/Export Information:	Import/Export text files and spreadsheet data.

Description/Purpose:

Tool takes as input a description of a system expressed graphically (using  STELLA) or using the MicroWorlds simulation language.  MicroWorlds Creator creates simulations and provides an effective user interface; mainly targeted for workshop/training use - e.g., for learning about software project dynamics and understanding software project management.

STSC Process Technologies Product Sheet Version 2.0b

ObjectMaker by Mark V Systems

Product Information:

Version Number:		2.10.26

Release Date:			August 1993

Frequency of Updates:	twice / year

Date of First Release: 	August 1992

Number Sold/Released:	2000+

Pricing:

Single User Price: 	$5000

Site License:     	Call for info

Multi-Copy Price: 	Call for info

Maintenance Price: 	15% of cost

Contact Information:

In business since: 	1974

Address:	16400 Ventura Blvd #303

Encino, CA  91436

Point of Contact: 	Marketing

Phone Number:	818-995-7671

Fax Number:		818-995-4267

E-mail Address: info@markv.com

objectmaker@markv.com

Customer Support :  support@markv.com

Process Technology Areas/Capabilities:

Behavioral modeling	        Code generation	Data Modeling

Object Modeling	        Process Modeling	Reverse Engineering

Intended Application Area:

Technical	        Real-time	Commercial

Intended Users of Technology/Tool:

Software Developers	        Software Managers	Process Engineers

Primary Methodology Base:

ADARTS	        Booch	Coad-Yourdon

Data Flow Diagram	        DeMarco-Yourdon	E-R Diagram

State Chart	        Firesmith	Gane-Sarson

Hatley-Pirbhai	        Petri Net	Shlaer-Mellor

Configurations:

Platform(s):   DEC RISC, DG Aviion, HP9000, IBM RS/6000, PC, Macintosh

MIPS RISC, Motorola 88000, Intel X86

Memory Requirements:

Data Import/Export Information:

Description/Purpose:

ObjectMaker is a software development workbench using a variety of object-oriented methods and supporting analysis, design, code generation, and reverse engineering.  All ObjectMaker editors and method definitions are integrated through a common semantic  repository based on PCTE.  Provides full online checking for completeness and consistency against the method design rules as designs are entered.  ObjectMaker's rule-based architecture allows notations, semantics, code generation, methods, menus, and accelerator keys to be tailored to match language, industry and company-specific requirements.  Works with language modules in Adagen (Ada) and Cgen (C and C++) which also provides code generation and reverse engineering.

STSC Process Technologies Product Sheet Version 2.0b

Objectory by Objective Systems

Product Information:

Version Number:		3.3.1

Release Date:			May 1, 1993

Frequency of Updates:	~ 12 mos.

Date of First Release: 	Sept., 1990

Number Sold/Released:	approx. 300

Pricing:

Single User Price: 	Range from 4-10K$

Site License:		Negotiable

Multi-Copy Price:	Discount up to 30%

Upgrade Costs:	Inc w/ maint cost

Maintenance Price:	20% of license fee/yr

Contact Information:

In business since: 		1987

Date research began:		1987

Address:	Box 1128

S-16422 Kista, Sweden

Phone Number: +46 8 703 45 30

Point of Contact: 	Sten Jacobson

Phone Number:	+46 8 703 45 84

Fax Number:		+46 8 751 30 96

E-mail Address:	sten@os.se

Electronic Bulletin Board:   mseru@os.se

Customer Support :             support@os.se

Process Technology Areas/Capabilities:

Definition	Process Examples	Process-Driven Framework

Modeling	Asset Library	Process-Driven SEE

Intended Application Area:

MIS                    Technical                   Real-time	Enterprise Modeling

Intended Users of Technology/Tool:

Software Developers

Primary Methodology Base:

Object-oriented                  Vose Jacobson Method

Configurations:

Platform(s):	DOS/Windows, SUN/Sparc/Soloris, IBM/RS6000 AIX, 	DEC/Ultrix, HP/Unix

Memory Requirements:	approx 16 MB recommended on all platforms

Data Import/Export Information:    Import/export facility using file mechanism

Description/Purpose:

Objectory is a development process and a CASE tool (Objectory Support

Environment) for the development of object-oriented software systems.  It is

an entire life cycle covering process.  The tool supports OOA and OOD.  The

Objectory process and tool has been used by several organizations for software

process modeling.

STSC Process Technologies Product Sheet Version 2.0b

PLAN/1 by Andersen Consulting

Product Information:

Version Number:

Release Date:

Frequency of Updates:

Date of First Release: 	1990

Number Sold/Released:	50

Pricing:

LAN Version: 	$7000

Maintenance Price:	15% of purchase

(Call for all other pricing information)

Contact Information:

Address:	69 West Washington St.

Chicago, IL 60602

Point of Contact:

Phone Number:	(312) 580-0069

Fax Number:		(312) 507-2548

Process Technology Areas/Capabilities :

Definition	Asset Library	Process-Driven Framework

Modeling	Process Examples	Process-Supported SEE

Intended Application Area:

MIS                                             Technical

Intended Users of Technology/Tool:

Software Developers                    Process Engineers

Primary Methodology Base:

Methodology Independent

Configurations:

Platform(s):		IBM PS/2 or compatible (Windows 3.0, OS/2 PM)

Memory Requirements:	12-16 MB RAM	24MB free on disk

(40MB disk space for an OS/2 work station)

Data Import/Export Information:

Description/Purpose:

PLAN/1 is an integrated toolset that supports strategic information planning and engineering.

STSC Process Technologies Product Sheet Version 2.0b

ProCAP by Knowledge Based Systems International

Product Information:

Version Number:		2.0

Release Date:

Date of First Release: 	6/30/92 (AI3)

Number Sold/Released:	not avail.

Pricing:

Single User Price:      	$4995

Site License:			Call for info

Multi-Copy Price:       	Call for info

Contact Information:

For vendor: in business since: 	1988

Address:   Knowledge Based Systems, Inc.

2746 Longmire Drive

College Station, TX 77845

Point of Contact: 	Sally Penick

Phone Number:	(409) 260-5274

Fax Number:		(409) 260-1965

E-mail Address:

Electronic BBS:	(409) 260-4606

Customer Support :      William Bosshard

Process Technology Areas/Capabilities:

Definition	Process Examples	Modeling

Asset Library

Intended Application Area:

Technical	MIS	Real-time

Intended Users of Technology/Tool:

Software Developers               Software Managers             Process Engineers

Primary Methodology Base:

IDEF	Behavior-oriented

Configurations:

Platform(s):	PC's running Windows

Memory Requirements:	2Meg

Data Import/Export Information:	ASCII Text File

Description/Purpose:

ProCAP is a windows tool supporting the IDEF3 method, both Process Flow and Object State Transition Network diagrams.  Like the other KBSI tools, ProCAP uses SmartDraw for worry free generation of diagrams.  Process flows allow the user to identify the temporal flows in a process as well as the logical flows.

STSC Process Technologies Product Sheet Version 2.0b

ProCEDE by International Software Systems, Inc.

Product Information:

Version Number:		Not Released

Release Date:  		May 1994

Frequency of Updates:	semiannually

Date of First Release: 	May 1994

Number Sold/Released:	not available

Pricing:

Call for Price Information

Contact Information:

In business since: 	1977

Date research began:

Address:  	9430 Research Boulevard

Echelon IV, Suite 250

Austin, TX   78759

Point of Contact: 	Dr. Cecil Martin

Phone Number:	(512)338-5729

Fax Number:		(512)338-5757

E-mail Address:	marketing@issi.com

Electronic BBS:  Wrld Wide Web URL:  http:

//sdio.issi.com/issi/issi-home_page.html

Cust. Support :  ProSLCSE-support@issi.com

Process Technology Areas/Capabilities:

Definition	Modeling	Process Examples

Asset Library

Intended Application Area:

Technical	MIS	Real-time

Other:  General usage by anyone defining processes

Intended Users of Technology/Tool:

Software Developers    Software Managers     Process Engineers

Other: Coordinators, Managers of any work-flow process

Primary Methodology Base:

Process Language (Graphical)	Methodology Independent

Other:  Visual Process Modeling Language and Integrated Improvement Methodology

Configurations:

Platform(s):     Sun SPARCstations running SunOS 4.1.x

Memory Requirements:     Not Yet Available

Data Import/Export Information:     Available upon request

Description/Purpose:

ProCEDE, a process-engineering development environment, is an integrated toolset supporting the definition and refinement of process models, infrastructure models, product definitions models, and project definition models.  The integrated tools provide graphical editing of a common, object-based model.  After defining processes, infrastructure, and products, a project manager can define an executable project by associating process models with infrastructure, and products, a project manager can define an executable project by associating process models with infrastructure and product models.  Personnel and other specific resources can be assigned to tasks.  A planned schedule for each task can also be specified.  In the future, ProCEDE will support the simulation of the project model.

STSC Process Technologies Product Sheet Version 2.0b

Process Weaver by Cap Gemeni

Product Information:

Version Number:		2.0

Release Date:			March 1994

Frequency of Updates:	Quarterly

Date of First Release: 	May 1992

Number Sold/Released:	100

Pricing:

Contact vendor for pricing information.

Contact Information:

For vendor: in business since: 	1972

For R&D: date research began:	1987

Address:	Cap Gemini America

1114 Avenue of the Americas

New York, NY 10036

Point of Contact: 	Larry Proctor

Phone Number:	(915) 947-7183

Fax Number:		(915) 942-9496

Process Technology Areas/Capabilities:

Assessment	Metrics Support	Process-Driven Framework

Definition	Process Examples	Process-Driven SEE

Modeling	Reusable Process Activity Objects	Enactment

Intended Application Area:

Technical	              MIS	Real-time

Any type of process which is repeatable and has defined steps

Intended Users of Technology/Tool:

Software Developers             Software Managers	Process Engineers

Primary Methodology Base:

Weaver uses a WBS breakdown structure to represent the overall methodology.

It uses state models to control the enactment process and Petri Nets to control

the overall sequencing of processing.

Configurations:

Platform(s): 	Sun 4, RS6000 (with or without Workbench), HP 9000/700

DEC, Windows 3.1 Client

Memory Requirements:   32 MB

Data Import/Export Information:

Description/Purpose:

Process Weaver is a framework in which process enactment models can be implemented.  Weaver allows the user to define the process using a hierarchical, top down approach using the Method Editor to input WBS structures.  The tool then uses state models to control the enactment process and Petri Nets to control the overall sequencing of the processing.

STSC Process Technologies Product Sheet Version 2.0b

ProcessMaker by Mark V Systems

Product Information:

Version Number:		2.1

Release Date:  		August 1993

Frequency of Updates:	4-6 mos.

Date of First Release: 	August 1993

Number Sold/Released:	25

Pricing:

Single User Price: 	$5000

Site License:    	Call for info

Multi-Copy Price: 	Call for info

Upgrade Costs: 	Call for info

Maintenance Price: 	15% of cost

Contact Information:

In business since: 	1974

Address:	16400 Ventura Blvd #303

Encino, CA  91436

Point of Contact: 	Marketing

Phone Number:	818-995-7671

Fax Number:		818-995-4267

E-mail Address:	info@markv.com

Customer Support :	support@markv.com

Process Technology Areas/Capabilities:

Definition	Process Examples	Customizable

FUTURE: 	Modeling	Enactment

Metrics Support	Asset Library

Intended Application Area:   All

Intended Users of Technology/Tool:

Software Developers         Software Managers            Process Engineers

Decision Support

Primary Methodology Base:

Process Language*     	Structured Analysis	Petri Nets

Entity Process Modeling	IDEF	Object-oriented

Behavior-oriented       	Methodology Independent

* Generates process language (or template of script) from graphic model for other tools

Configurations:

Platform(s):     Most UNIX, VMS and PC Windows

Memory Requirements:     UNIX/VMS - 12 MB              PC - 4 MB

Data Import/Export Information:     Under Development

Description/Purpose:

ProcessMaker is a rule-driven repository-based graphical process modeling tool supporting more than 12 notations and techniques.  Additional methods/notations are being added, and all are customizable.  Extensions to generate templates/script for process analysis and enactment tools are under development.

STSC Process Technologies Product Sheet Version 2.0b

ProEDITOR by International Software Systems, Inc.

Product Information:

Version Number:		2.2

Release Date:  		Feb 7, 1994

Frequency of Updates:	semiannually

Date of First Release: 	May 1993

Number Sold/Released:	not available

Pricing:

Call for Price Information

Contact Information:

In business since: 	1977

Date research began:

Address:  	9430 Research Boulevard

Echelon IV, Suite 250

Austin, TX   78759

Point of Contact: 	Dr. Cecil Martin

Phone Number:	(512)338-5729

Fax Number:		(512)338-5757

E-mail Address:	marketing@issi.com

Electronic BBS:  Wrld Wide Web URL:  http:

//sdio.issi.com/issi/issi-home_page.html

Cust. Support :  ProSLCSE-support@issi.com

Process Technology Areas/Capabilities:

Definition	Modeling	Process Examples

Asset Library

Intended Application Area:

Technical	MIS	Real-time

Other:  General usage by anyone defining processes

Intended Users of Technology/Tool:

Software Developers    Software Managers     Process Engineers

Other: Coordinators, Managers of any work-flow process

Primary Methodology Base:

Process Language (Graphical)	Methodology Independent

Other:  Visual Process Modeling Language and Integrated Improvement Methodology

Configurations:

Platform(s):     Sun SPARCstations running SunOS 4.1.x; soon available on PC's

Memory Requirements:     24 MB RAM, 50MB free disk space, 120 MB swap space

Data Import/Export Information:     ASCII file format available upon request

Description/Purpose:

ProEDITOR is a structure-oriented graphical editor for Visual Process Modeling language (VPML).  VPML is an intuitive, graphical language that allows users to express their business processes easily.  VPML shows, in varying levels of detail, the tasks in a process, resources that are allocated to tasks, products that are used and created by tasks, and the order in which tasks are performed.  ProEDITOR understands the rules for interconnecting VPML components and uses these rules to help users produce correct VPML process models.

STSC Process Technologies Product Sheet Version 2.0b

ProGUIDE by International Software Systems, Inc.

Product Information:

Version Number:		2.2 beta

Release Date:  		Feb 1994

Frequency of Updates:	semiannually

Date of First Release: 	May 1993

Number Sold/Released:	not available

Pricing:

Call for Price Information

Contact Information:

In business since: 	1977

Date research began:

Address:  	9430 Research Boulevard

Echelon IV, Suite 250

Austin, TX   78759

Point of Contact: 	Dr. Cecil Martin

Phone Number:	(512)338-5729

Fax Number:		(512)338-5757

E-mail Address:	marketing@issi.com

Electronic BBS:  Wrld Wide Web URL:  http:

//sdio.issi.com/issi/issi-home_page.html

Cust. Support :ProSLCSE-support@issi.com

Process Technology Areas/Capabilities:

Enactment	Process-Driven Framework	Metrics Support

Intended Application Area:

Technical	MIS	Real-time

Intended Users of Technology/Tool:

Software Developers    Software Managers     Process Engineers

Other: Coordinators, Managers of any work-flow process

Primary Methodology Base:

Process Language (Graphical)	Methodology Independent

Other:  Visual Process Modeling Language and Integrated Improvement Methodology

Configurations:

Platform(s):     Sun SPARCstations running SunOS 4.1.x

Memory Requirements:     24 MB RAM, 50 MB free disk space, 120 MB swap space

Data Import/Export Information:     Available upon request

Description/Purpose:

ProGUIDE executes the projects defined using ProCEDE.  During project execution, the processes incorporated into the project are enacted.  Each person is provided a work agenda which is kept up-to-date by ProGUIDE.  The work agenda shows which tasks are pending, ready, active, and completed.  Only work tasks assigned to a person show up on his/her work agenda.  By selecting a task on the work agenda, a task card appears providing graphical guidance.  It shows the product inputs and outputs from the task, and permits launching of tools to manipulate products.  When a task is marked completed. ProGUIDE notifies the people assigned to the next task in the process that the completed product is ready.

STSC Process Technologies Product Sheet Version 2.0b

ProSIM by Knowledge Based Systems International

Product Information:

Version Number:		2.0

Release Date:

Date of First Release: 	6/30/92 (AI3)

Number Sold/Released:	not avail.

Pricing:

Single User Price:      	$5995

Site License:			Call for info

Multi-Copy Price:       	Call for info

Contact Information:

For vendor: in business since: 	1988

Address:   Knowledge Based Systems, Inc.

2746 Longmire Drive

College Station, TX 77845

Point of Contact: 	Sally Penick

Phone Number:	(409) 260-5274

Fax Number:		(409) 260-1965

E-mail Address:

Electronic BBS:	(409) 260-4606

Customer Support : 	William Bosshard

Process Technology Areas/Capabilities:

Definition	Process Examples	Modeling

Asset Library

Intended Application Area:

Technical	MIS	Real-time

Intended Users of Technology/Tool:

Software Developers               Software Managers             Process Engineers

Primary Methodology Base:

IDEF	Behavior-oriented

Configurations:

Platform(s):	PC's running Windows

Memory Requirements:	2Meg

Data Import/Export Information:	ASCII Text File

Description/Purpose:

ProSIM is an enhanced version of ProCAP that allow the model to be placed

into simulation using AT&T Istel's Witness® simulation package for Windows_.

STSC Process Technologies Product Sheet Version 2.0b

ProSLCSE by International Software Systems, Inc.

Product Information:

Version Number:		2.2

Release Date:  		By Parts

Frequency of Updates:	semiannually

Date of First Release: 	May 1993

Number Sold/Released:	not available

Pricing:

Call for Price Information

ProSLCSE tool suite product line is

priced separately

Contact Information:

In business since: 	1977

Date research began:

Address:  	9430 Research Boulevard

Echelon IV, Suite 250

Austin, TX   78759

Point of Contact: 	Dr. Cecil Martin

Phone Number:	(512)338-5729

Fax Number:		(512)338-5757

E-mail Address:	marketing@issi.com

Electronic BBS:  Wrld Wide Web URL:  http:

//sdio.issi.com/issi/issi-home_page.html

Cust. Support :ProSLCSE-support@issi.com

Process Technology Areas/Capabilities:

Definition	Modeling	Enactment

Metrics Support	Process Examples	Asset Library

Process-Driven Framework	Process-Driven SEE

Intended Application Area:

Technical	MIS	Real-time

Intended Users of Technology/Tool:

Software Developers	Software Managers	Process Engineers

Coordinators and Managers of any work-flow process

Primary Methodology Base:

Process Language (Graphical)	Methodology Independent

Visual Process Modeling Language and Integrated Improvement Methodology

Configurations:

Platform(s):     Sun SPARCstations running SunOS 4.1.x; soon available on PC's

Memory Requirements:    see each product separately

Data Import/Export Information:     Data Import/Export provided by environment

Description/Purpose:

ProSLCSE is a product line of process tools, process-directed environments, and process services.  ProSLCSE helps organizations capture, guide, evaluate, and optimize their business processes.  ProSLCSE tools are structured so that users can incrementally adopt process engineering as part of their business process.  The integrated ProSLCSE products form a process-directed framework which can be transformed into a customized process-directed environment.  The ProSLCSE product line includes ProEDITOR, ProCEDE, ProGUIDE, ProGUARD, ProMANAGER, and ProEXTENDER.

ProEDITOR is a syntax-directed editor for the graphical Visual Process Modeling Language (VPML).  Using ProEDITOR, graphical process models can be defined and refined.  ProCEDE, a process engineering development environment, provides complete support for enterprise modeling, project definition, and simulation.  ProGUIDE enacts processes defined with ProCEDE.  ProGUARD provides process control thereby insuring the integrity of the process and the products.  ProMANAGER gives the project manager up-to-date status and metrics on  process enactment.  ProEXTENDER includes tools that support the transformation of the ProSLCSE framework into a customized process-directed SEE.

STSC Process Technologies Product Sheet Version 2.0b

ProTEM by Software Consultants International Ltd.

Product Information:

Version Number:		1.01

Release Date:			Dec. 1992

Frequency of Updates:	Min. 1/year

Date of First Release: 	Dec. 1992

Number Sold/Released:	10

Pricing:

Single User Price: 	$495 per copy

Site License:		Negotiable

Multi-Copy Price: 	Negotiable

Upgrade Costs:	First upgrade free

GSA Price:		Not yet established

Maintenance Price: 	Now included, but

subject to revision

Contact Information;

In business since: 		1982

Date research  began: 	1987

Shipping Addr:13812 SE 240th St.

Kent, WA 98042-3321

Mailing Addr:	 PO Box 5712

Kent, WA 98064-5712

Point of Contact: 	Lawrence Peters

Phone Number:	(206) 631-4212

Fax Number:		(206) 631-4212

E-mail Address:	None

Electronic Bulletin Board:  None yet

Customer Support : 	(216) 631-4212

Process Technology Areas/Capabilities:

Definition	Process Improvement 	Metrics Support

Modeling	Process Examples

Intended Application Area:           All

Intended Users of Technology/Tool:

Software Developers               Software Managers                Process Engineers

Primary Methodology Base:   Petri Nets

Configurations:

Platform(s):  PC's, 386DX (min), 486DX (recommended) with MSWindows

3.0 or above, VGA graphics recommended, EGA minimum

Memory Requirements:4 MB RAM (min)

Data Import/Export Information:	Not applicable

Description/Purpose:

Addresses the general problem of modeling and simulating process.  This includes software development and engineering activities.  The key to the approach used in ProTEM is the masking of the complexities associated with Petri net like modeling systems so that any user who knows more about the process he/she wishes to simulate than advanced mathematics will be able to obtain meaningful results.  Using ProTEM, one can get an estimate of the flow time associated with the modeled process as well as its potential for violation of one or more policy requirements.

STSC Process Technologies Product Sheet Version 2.0b

RDD-100 by Ascent Logic

Product Information:

Version Number:		4.0

Release Date:  		3Q93

Frequency of Updates:

Date of First Release:

Number Sold/Released:

Pricing:

$2,000 for stand-alone license of the System Browser to $39,000 for a networked license of the System Designer.

Contact Ascent Logic for quotations to specific configurations.

Contact Information:

In business since:

Date research began:

Address:  180 Rose Orchard Way, Ste 200

San Jose, CA  95134

Point of Contact:

Sales:    Pete Casoni (408)943-0630 x136

Tech Support:  Jon Tatar (800)654-4733

Phone Number:	(408)943-0630

Fax Number:		(408)943-0705

E-mail Address:	casoni@alc.com

Customer Support :	tatar@alc.com

Process Technology Areas/Capabilities:

Definition	Data Modeling	Simulation

Report Gen	Metrics Support	Assessment

Process Examples

Intended Application Area:

All areas of complex system design and simulation

Intended Users of Technology/Tool:

Software Developers  	         Software Managers	Process Engineers

Integration Engineers  	         Test Engineers	System Managers

Primary Methodology Base:

Behavior Diagrams	    Process Dependency Diagrams	Data Flow Diagrams

Data Model Diagrams     Process Narrative	Backus-Naur Notation

E-R Model	    State Transition Diagrams	IDEF

Process Language

Configurations:

Platform(s):     Mac, Decstation, HP 9000, IBM RISC, Sun, PC

Memory Requirements:     24 MB RAM Minimum,  60MB Disk Space

Data Import/Export Information:  Via ASCII files.  Bridge to CADRE's Teamwork.

Description/Purpose:

RDD-100_ is a technology aimed at automation of the system description and design process.  RDD-100_ is targeted at analysis, design, and description of complex systems including processes and real-time distributed embedded systems.  Applications range from system lifecycle activities to change impact analysis and reverse engineering and re-engineering of mature systems.  RDD-100 may be applied to both descriptive modeling of deliverable systems and the processes for their production.  RDD included an adaptive Systems Engineering Method, a Systems Engineering Notation and a software tool to automate the process.  RDD-100_ support concurrent engineering and rapid application development.

STSC Process Technologies Product Sheet Version 2.0b

S**4 by MicroWorlds, Inc.

Product Information:

Version Number:		3.3L

Release Date:			5/92

Frequency of Updates:	Quarterly

Date of First Release: 	1/91

Number Sold/Released:	150

Pricing:

Single User Price: 	First copy: $4900

Subsequent copies: 	$1950

Upgrade Costs:  	1st year free

GSA Price:		--------

Maintenance Price: 	No charge

Contact Information:

For vendor: in business since: 	1988

For R&D: date research began:

Address:	47 Third St., Suite 200

Cambridge, MA 02141

Point of Contact: 	Elizabeth Peterson

Phone Number:	617) 225-0025

Fax Number:		617) 225-0028

E-mail Address:	AppleLink: D3640

Electronic Bulletin Board:	none

Customer Support : 	(617) 225-0025

Process Technology Areas/Capabilities:

Modeling	Process Examples              Process-Driven Framework

Metrics Support	Asset Library

Intended Application Area:

Technical	Real-time

MIS	Project Management/Strategy

Intended Users of Technology/Tool:

Model Developers                    Software Managers

Primary Methodology Base:

System Dynamics                    Behavior-oriented

Configurations:

Platform(s):	Macintosh II or greater

Memory Requirements: 	2 MB.

Data Import/Export Information: Import & export text files and spreadsheet data;

connect to HyperCard, SuperCard, Wingz, etc.

Description/Purpose:

Tool takes as input a description of a system expressed graphically (using STELLA) or using the MicroWorlds simulation language.  S**4 allows the user to create his own interface.  In addition, S**4 lets the modeler examine the simulation by allowing him to interactively walk his way down into the model.

STSC Process Technologies Product Sheet Version 2.0b

SIMprocess by CACI

Product Information:

Version Number:		4.2

Release Date:  		8/92

Frequency of Updates:	 2/yr

Pricing:

Single User Price:  $9,500

Contact Information:

In business since:	1962

Date research began:	1962

Address:

1100 N. Glebe Rd.

Arlington, VA 22201

Point of Contact: 	Ron Flauaus

Phone Number:	703-841-7902

Fax Number:		703-528-4196

Process Technology Areas/Capabilities:

Metrics Support                    Modeling

Intended Application Area:

MIS                                     Business Process Engineering

Intended Users of Technology/Tool:

Process Engineers               Systems Analysts

Primary Methodology Base:

IDEF

Configurations:

Platform(s):	DOS, MAC, UNIX

Memory Requirements:

Data Import/Export Information:  Accepts import data to specify a model; also exports 			       simulation results to LOTUS and EXCEL

Description/Purpose:

SIMprocess is a modeling/simulation tool for business process engineering efforts.  With SIMprocess, a user may build a graphical model of "organization design" and then verify the design through simulation.

STSC Process Technologies Product Sheet Version 2.0b

STATEMATE by I-Logix, Inc.

Product Information:

Version Number:		5.0

Release Date:			Aug 1993

Frequency of Updates:	2/year

Date of First Release: 	1987

Number Sold/Released:	1300

Pricing:

Single User Price: 	$25,000 and up

Site License:		Call for information

Multi-Copy Price:	Call for information

Upgrade Costs:	Inc in spprt contrct

GSA Price:		N/A

Maintenance Price: 	15%

Contact Information:

In business since: 	1986

Address:	Three Riverside Dr.

Andover Research Park

Andover, MA

01810

Point of Contact: Mike F. Kelly

Phone Number:	(508) 682-2100

Fax Number:		(508) 682-5995

E-mail Address:      mike@ilogix.com

Electronic Bulletin Board:	N/A

Customer Support : 	(508) 682-4884

Process Technology Areas/Capabilities:

Assessment	                 Process Examples	Definition

Process-Driven Framework    Modeling	Process-Supported SEE

Intended Application Area:

Real Time

Intended Users of Technology/Tool:

Software Developers    	Software Managers     	Process Engineers

Systems Engineers     	Hardware Engineers

Primary Methodology Base:

Process Language	SADT/Struct Analysis	Entity Process Modeling

Behavior-oriented	Methodology Independent

Configurations:

Platform(s):	SUN, DEC (VMS or Ultrix), IBM RS6000, Apollo, HP

Memory Requirements:	      12M minimum, 16M recommended

Data Import/Export Information:    Complete access to 100% ASCII Repository,

including graphical information.  Ability to extract info and

create custom templates; 2167A templates provided.  Support

for Interleaf as well as postscript, NROFF, TROFF, etc.

Description/Purpose:

Statemate allows for the graphical description and analysis of any reactive system, whether it is a process, a computer or an ASIC.  If has been used for process modeling successfully by SEI, IBM, Motorola and Boeing.

STSC Process Technologies Product Sheet Version 2.0b

Synergy by CASE Methods Development Corporation

Product Information:

Version Number:		2.0

Release Date:			April 1, 1993

Frequency of Updates:	6 months

Date of First Release: 	August, 1991

Pricing:

Contact vendor for pricing information

Contact Information:

In business since: 	1988

Date research began:	1988

Address:	100 North Central Exprswy

Suite #710

Richardson, Texas 75080

Phone Number: 	(214) 437-9700

Customer Support : 	(214) 644-8173

Process Technology Areas/Capabilities:

Assessment	Metrics Support	Process-Driven Framework

Definition	Process Examples	Process-Supported SEE

Modeling	Asset Library	Process-Driven SEE

Intended Application Area:

All

Intended Users of Technology/Tool:

Primary Methodology Base:

Can be customized to support IDEF            ANY METHODOLOGY BASE?

Configurations:

Platform(s):	Windows 3.1; specific workstations can be configured

Memory Requirements:	8 MB RAM, 20 MB Disk Space

Data Import/Export:	Import methodology or document text, e.g., standards

Export WBS for project management and text

Description/Purpose:

Product is primarily designed to improve the development process used for the

delivery of application systems.  The focus is on quality, team communication, and refinement of process definitions, standards and procedures.

STSC Process Technologies Product Sheet Version 2.0b

SynerVision by Hewlett Packard

Product Information:

Frequency of Updates:	9-12 months

Date of First Release: 	6/93

Number Sold/Released:	internal/Beta

Pricing:

Single User Price: 	$3950

GSA Price:		approx 35% discount

Maintenance Price: 	$593/year

Contact Information:

In business since: 	1989

Date research began:	1989

Address:    SW Engineering System Div.

3404 E. Harmony Rd.

Ft. Collins, CO 80525-9599

Phone Number: 	(303) 229-3800

Point of Contact: 	Charlie Doucot

Phone Number:	(617) 221-5191

Process Technology Areas/Capabilities:

Assessment	Metrics Support	Process-Driven Framework

Definition	Enactment	Process-Supported SEE

Modeling	Asset Library	Process-Driven SEE

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers              Software Managers                  Process Engineers

Primary Methodology Base:

Rule-based	Methodology Independent

Configurations:

Platform(s):  	HP

Memory Requirements:

Data Import/Export Information:

Description/Purpose:

Synervision is a process enactment product useful at all levels of process maturity - useful for task management, process management, communication/teamwork, and metrics collection and reporting (real-time)

STSC Process Technologies Product Sheet Version 2.0b

System Engineer Toolset by LBMS, Inc.

Product Information:

Version Number:		4.01A

Release Date:			1992

Frequency of Updates:

Date of First Release: 	1997

Number Sold/Released:	15,000

Pricing:

For pricing information, call :

(800)231-7515

Contact Information:

In business since: 	1977

Address:	LBMS, Inc.

1800 West Loop South

Suite 1800

Houston, TX 77027

Phone Number: 	713- 623-0414

1-800-231-7515

Process Technology Areas/Capabilities:

Definition	Enactment	Process-Driven Framework

Modeling	Process Examples	Process-Supported SEE

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers    	Software Managers     	Process Engineers

Primary Methodology Base:

Process Language		Structured Analysis	System Dynamics

IDEF		Petri Nets  	Entity Process Modeling

Rule-based		Object-oriented	Methodology Independent

Behavior-oriented

Configurations:

Platform(s):		     386/486 Intel Based

Memory Requirements:    8 MG Memory, 200 MG Hard Disk

Data Import/Export Information:

Description/Purpose:

An integrated, comprehensive family of products covering the full lifecycle: analysis, design, prototyping and construction. Systems Engineer supports requirements and analysis; Project Engineer provides support for defining and instantiating a process, either from LBMS systems engineering methods or your own.  The toolset shares an instantly accessible central workgroup database; each tool accesses information stored in the central workgroup database; changes to any  object are automatically rippled throughout the whole design.

STSC Process Technologies Product Sheet Version 2.0b

System Factory Project by USC

Product Information:

Version Number:		6/92

Release Date:			June, 1992

Frequency of Updates:	6 months

Date of First Release:   	Dec. 1991

Number Sold/Released:	6

Pricing:

Distributed to research sponsors per

contracted terms

Contact Information:

Date research began:	1981

Address:  Decision Systems Department

University of Southern California

Los Angeles, CA 90089-1421

Point of Contact: 	Dr. Walt Scacchi

Phone Number:	(213) 740-4782

Fax Number:		(213) 740-8494

E-mail Address:   scacchi@pollux.usc.edu

Process Technology Areas/Capabilities:

Assessment	Metrics Support	Process-Driven Framework

Definition	Process Examples	Process-Supported SEE

Modeling	Asset Library	Process-Driven SEE

Enactment	Visualization 	Analysis

Simulation 	Process Integration

Intended Application Area:

Technical                                  MIS                                    Real-time

Intended Users of Technology/Tool:

Software Developers           Software Managers            Process Engineers

Can be customized to any defined user role

Primary Methodology Bases:

Process Language	Entity Process Modeling	Rule-based

Object-oriented		Behavior-oriented	Knowledge-Based

Methodology Independent (therefore can also support structured analysis, system dynamics, Petri nets and IDEF though these are considered to be  weaker methodologies by the System Factory developers)

Configurations:

Platform(s): SUN SPARC Stations-2, Knowledge-Craft from Carnegie Group

Memory Requirements:	24 MB, 75 MB swap/disk

Data Import/Export Information:	Compatible with heterogeneous, distributed

data and tool repositories

Description/Purpose:

We have developed and continue to evolve and deliver a configurable environment of software process engineering technologies for large-scale development applications.  Our technology is being transferred into commercial products at this time.  A largest project entails the development of a prototype process-driven environment for use in the B-2 Division at Northrop, Inc., in Peco Rivera, CA.  Additional reference material is available.

STSC Process Technologies Product Sheet Version 2.0b

Teamwork/IM SQL by Cadre Technologies, Inc.

Product Information:

Version Number:		5.0

Release Date:  		July 1993

Frequency of Updates:	9-18 months

Date of First Release: 	1985

Number Sold/Released:	not available

Pricing:

Single User Price: 	$2125

Site License:     	n/a

Multi-Copy Price: 	all vendor

Upgrade Costs:	included in yearly

maintenance

GSA Price:		$1309

Maintenance Price: 	$318.75 per year

Contact Information:

In business since: 	1982

Address:	222 Richmond St.

Providence, RI 02903

Point of Contact: 	Jeff Jenks

Phone Number:	408-562-0106

Fax Number:		408-987-5775

Electronic Bulletin Board: Available to -

Customer Support :	     customers only

Process Technology Areas/Capabilities:

Modeling	Metrics Support

Intended Application Area:   All

Intended Users of Technology/Tool:

Software Developers                      Software Managers

Primary Methodology Base:

Structured Analysis                   Entity Process Modeling

Configurations:

Platform(s):   Sun (SunOS & Solaris, HP (DOMAIN & HP-UX), IBM AIX, DEC,

Ultrix &VAX-VMS, Silicon Graphics, Intel (Solaris & OS/2)

Memory Requirements:     Varies depending on configuration

Data Import/Export Information: Cadre supports CDIF import & export capabilities.

Description/Purpose:

Teamwork/IM SQL supports modeling the entity relationships and attributes of

all application data at the conceptual level.  Teamwork/IM SQL's Entity-

Relationship editor provides for the graphical capture and editing of entity-

relationship diagrams based on Chen's notation.  Teamwork/IM SQL has

consistency and completeness checking.  Data definitions, associated with

entities and relationships, can be automatically generated with user-configurable

physical and logical attributes.  Teamwork/IM SQL also supports SQL DDL

generation for Oracle 5, Sybase, and ANSI SQL.

STSC Process Technologies Product Sheet Version 2.0b

Teamwork/OOA, Teamwork/OOD by Cadre Technologies, Inc.

Product Information:

Version Number:		5.0

Release Date:  		July 1993

Frequency of Updates:	9-18 months

Date of First Release: 	1989 (OOA)

1991 (OOD)

Number Sold/Released:	not available

Pricing:

Single User Price: 	$4800 (OOA)

$3200 (OOD)

Site License:     	n/a

Multi-Copy Price: 	all vendor

Upgrade Costs:	included in yearly

maintenance

GSA Price:		not available

Maintenance Price: 	$720 /yr (OOA)

$480 /yr (OOD)

Contact Information:

In business since: 	1982

Address:	222 Richmond St.

Providence, RI 02903

Point of Contact: 	Jeff Jenks

Phone Number:	408-562-0106

Fax Number:		408-987-5775

Electronic Bulletin Board: Available to -

Customer Support :	    customers only

Process Technology Areas/Capabilities:

Modeling	Metrics Support

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers                      Software Managers

Primary Methodology Base:

Structured Analysis

Configurations:

Platform(s):   Sun (SunOS & Solaris, HP (DOMAIN & HP-UX), IBM AIX, DEC,

Ultrix &VAX-VMS, Silicon Graphics, Intel (Solaris & OS/2)

Memory Requirements:     Varies depending on Configurations

Data Import/Export Information: Cadre supports CDIF import & export capabilities.

Description/Purpose:

Teamwork/OOA and Teamwork/OOD provide complete support for the

Shlaer-Mellor object-oriented methodology, including support for all diagrams

and notation.

STSC Process Technologies Product Sheet Version 2.0b

Teamwork/RT by Cadre Technologies, Inc.

Product Information:

Version Number:		5.0

Release Date:  		July 1993

Frequency of Updates:	9-18 months

Date of First Release: 	1985

Number Sold/Released:	not available

Pricing:

Single User Price: 	$1925

Site License:     	n/a

Multi-Copy Price: 	all vendor

Upgrade Costs:	inc w/ maintenance

GSA Price:		$1309

Maintenance Price: 	$288.75 per year

Contact Information:

In business since: 	1982

Address:	222 Richmond St.

Providence, RI 02903

Point of Contact: 	Jeff Jenks

Phone Number:	408-562-0106

Fax Number:		408-987-5775

Electronic Bulletin Board:  Available to -

Customer Support :	   customers only

Process Technology Areas/Capabilities:

Modeling	Metrics Support

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers                      Software Managers

Primary Methodology Base:

Structured Analysis             Structured Dynamics                Behavior-Oriented

Configurations:

Platform(s):   Sun (SunOS & Solaris, HP (DOMAIN & HP-UX), IBM AIX, DEC,

Ultrix &VAX-VMS, Silicon Graphics, Intel (Solaris & OS/2)

Memory Requirements:     Varies depending on Configurations

Data Import/Export Information: Cadre supports CDIF import & export capabilities.

Description/Purpose:

Teamwork/RT gives designers the tools to deal with real-time issues of sequence,

timing, and control.  Teamwork/RT features a unified Control Flow Diagram/Data

Flow Diagram that allows extensions to the standard Teamwork/SA editor that

include control flows and control specification connectors.  Additionally,

Teamwork/RT includes State Transition Diagram and Matrix editors to create,

check, and edit Decision Tables, State Transition Diagrams, Process Activation

Tables, and State Event Matrices.

STSC Process Technologies Product Sheet Version 2.0b

Teamwork/SA by Cadre Technologies, Inc.

Product Information:

Version Number:		5.0

Release Date:  		July 1993

Frequency of Updates:	9-18 months

Date of First Release: 	1985

Number Sold/Released:	not available

Pricing:

Single User Price: 	$1925

Site License:     	n/a

Multi-Copy Price: 	all vendor

Upgrade Costs:	inc w/ maintenance

GSA Price:		$1309

Maintenance Price: 	$288.75 per year

Contact Information:

In business since: 	1982

Address:	222 Richmond St.

Providence, RI 02903

Point of Contact: 	Jeff Jenks

Phone Number:	408-562-0106

Fax Number:		408-987-5775

Electronic Bulletin Board: Available to -

Customer Support :	    customers only

Process Technology Areas/Capabilities:

Modeling	Metrics Support

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers                      Software Managers

Primary Methodology Base:

Structured Analysis

Configurations:

Platform(s):   Sun (SunOS & Solaris, HP (DOMAIN & HP-UX), IBM AIX, DEC,

Ultrix &VAX-VMS, Silicon Graphics, Intel (Solaris & OS/2)

Memory Requirements:     Varies depending on configuration

Data Import/Export Information:Cadre supports CDIF import & export capabilities.

Description/Purpose:

Teamwork/SA supports systems and software analysis.  It features an intelligent Data Flow Diagram editor based on standard Yourdon-DeMarco methods.  Teamwork/SA gives designers the ability to graphically construct, check, store, and edit Data Flow Diagrams complete with automatic generation of Process Specifications for textual detail and Data Dictionary entries that define data items.

STSC Process Technologies Product Sheet Version 2.0b

Teamwork/SD by Cadre Technologies, Inc.

Product Information:

Version Number:		5.0

Release Date:  		July 1993

Frequency of Updates:	9-18 months

Date of First Release: 	1985

Number Sold/Released:	not available

Pricing:

Single User Price: 	$1925

Site License:     	n/a

Multi-Copy Price: 	all vendor

Upgrade Costs:	inc w/ maintenance

GSA Price:		$1309

Maintenance Price: 	$288.75 per year

Contact Information:

In business since: 	1982

Address:	222 Richmond St.

Providence, RI 02903

Point of Contact: 	Jeff Jenks

Phone Number:	408-562-0106

Fax Number:		408-987-5775

Electronic Bulletin Board: Available to -

Customer Support :	     customers only

Process Technology Areas/Capabilities:

Modeling	Metrics Support

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers                      Software Managers

Primary Methodology Base:

Structured Analysis

Configurations:

Platform(s):   Sun (SunOS & Solaris, HP (DOMAIN & HP-UX), IBM AIX, DEC,

Ultrix &VAX-VMS, Silicon Graphics, Intel (Solaris & OS/2)

Memory Requirements:     Varies depending on Configurations

Data Import/Export Information:Cadre supports CDIF import & export capabilities.

Description/Purpose:

Teamwork/SD supports structured design techniques.  Teamwork/SD's design

tools, based on the Constantine method, support the graphical creation and

integration of structure charts, complete with module specifications and data

dictionary definitions.  Structure chart elements include modules of various types,

off-page connectors, module invocation lines, couples, iteration symbols,

transaction centers, and data-hiding modules.

STSC Process Technologies Product Sheet Version 2.0b

Teamwork/Sim by Cadre Technologies, Inc.

Product Information:

Version Number:		5.0

Release Date:  		July 1993

Frequency of Updates:	9-18 months

Date of First Release: 	1985

Number Sold/Released:	not available

Pricing:

Single User Price: 	$6900

Site License:     	n/a

Multi-Copy Price: 	all vendor

Upgrade Costs:	inc w/ maintenance

GSA Price:		$4629

Maintenance Price: 	$742.5 per year

Contact Information:

In business since: 	1982

Address:	222 Richmond St.

Providence, RI 02903

Point of Contact: 	Jeff Jenks

Phone Number:	408-562-0106

Fax Number:		408-987-5775

Electronic Bulletin Board: Available to -

Customer Support :	    customers only

Process Technology Areas/Capabilities:

Modeling	Metrics Support

Intended Application Area:   All

Intended Users of Technology/Tool:

Software Developers                      Software Managers

Primary Methodology Base:

Structured Analysis             Structured Dynamics                Behavior-Oriented

Configurations:

Platform(s):   Sun (SunOS & Solaris, HP (DOMAIN & HP-UX), IBM AIX, DEC,

Ultrix &VAX-VMS, Silicon Graphics, Intel (Solaris & OS/2)

Memory Requirements:     Varies depending on Configurations

Data Import/Export Information:Cadre supports CDIF import & export capabilities.

Description/Purpose:

Teamwork/Sim provides users the capability to simulate (in interactive or

batch mode) test cases against the Data and/or Control Flow Diagrams captured

during the analysis stage.  "Essential view" simulation will detect behavior and

performance problems independent of hardware resource considerations.

Additional insight into the system can be gained by "implementation view"

simulation which considers hardware resource issues such as contention,

pre-emption, prioritization, software to hardware mapping, and store access delays.

STSC Process Technologies Product Sheet Version 2.0b

Tool Builder by The Carriage House

Product Information:

Version Number:		1.3.2

Release Date:			May '93

Frequency of Updates:	one/year

Date of First Release: 	1989

Number Sold/Released:	100

Pricing:

Call for price information

Contact Information:

Address:	The Carriage House

28 Green Street

Newbury, MA 01951

Phone Number: 	(508) 463-0006 or

1-800 METACASE

Point of Contact: 	Kate McDonough

Phone Number:	(508) 463-0006

Fax Number:		(508) 462-9198

Process Technology Areas/Capabilities :

META CASE Tool for developing process support environments

Intended Application Area:

All

Intended Users of Technology/Tool:

Software Developers

Primary Methodology Base:

Entity Process Modeling 	Rule-based

Methodology Independent	The ToolBuilder Method

Configurations:

Platform(s):			SUN/HP/IBM and Digital VMS

Memory Requirements:	16MB

Data Import/Export Information:	Yes

Description/Purpose:

IPSYS ToolBuilder is an extremely powerful integrated CASE tool that is used to build other CASE tools supporting traditional, object oriented, or an organization's own method.  IPSYS has used Tool Builder to build a prototype process modeling environment as part of a European project.

STSC Process Technologies Product Sheet Version 2.0b

TurboCASE by Structsoft

Product Information:

Version Number:		4.22

Release Date:   		Apr 25, '93

Frequency of Updates:	Frequent

Date of First Release: 	Sept., 1989

Number Sold/Released:	approx 1400

Pricing:

Single User Price: 	$995

Site License:		Negotiable

Multi-Copy Price: 	10%-40% off

Upgrade Costs:	$0 - $149

GSA Price:		Not on the GSA list

Maintenance Price: 	None

Contact Information:

In business since: 	1984

Date research began:	1984

Address:	5416 156th Ave SE

Bellevue, WA 98006

Phone Number: 	(206) 644-9834

Point of Contact: 	Shang-cheng Chyou

Phone Number:	(206) 644-9834

Fax Number:		(206) 644-7714

E-mail Addr:	AppleLink: STRUCTSOFT

CompuServe: [76636, 254]

Customer Support: 	(206) 644-9834

Process Technology Areas/Capabilities:

Definition                               Modeling                               Process Examples

Intended Application Area:

Technical	Real-time

Intended Users of Technology/Tool:

Software Developers                Software Managers                   Process Engineers

Primary Methodology Base:

Structured Analysis    		Entity Process Modeling

Object-oriented			Behavior-oriented

Configurations:

Platform(s):	Macintosh

Memory Requirements:	2 MB

Data Import/Export Information:	Input/Export ASCII Information Bridge

between TurboCASE & Cadre's Teamwork

Description/Purpose:

TurboCASE is multi-window, multi-methodology supporting Macintosh CASE

tool.  It supports the traditional analysis and design techniques as well as the newer Object-Oriented Analysis and design methodologies.

STSC Process Technologies Product Sheet Version 2.0b

Virtual Software Factory by Virtual Software Factory, Ltd.

Product Information:

Version Number:		3.7.2

Release Date:			Dec. 1991

Date of First Release: 	1988

Number Sold/Released:	40

of Tool Builder.

Pricing:

Single User Price: $25K MWB (Tl Builder)

$11.5K AWB (Run Time)

Multi-Copy Price: 	$2K - 11.5 K

Maintenance Price: 	17.5% of price

Contact Information:

In business since: 	1985

Address:	8300 Boone Boulevard

Vienna, VA  22182

Point of Contact: 	Dr. Mel Selwood

Phone Number:	(703) 848-9282

Fax Number:		(703) 893-9508

E-mail Address:

Customer Support :	+44-202-297292

(UK Headquarters)

Process Technology Areas/Capabilities:

Assessment	Enactment	Process-Driven Framework

Definition	Process Examples	Process-Supported SEE

Modeling	Asset Library	Process-Driven SEE

Metrics Support (via external tools)

Intended Application Area:

Technical	MIS	Application-Specific Engineering

Real-time	Reverse Engineering	Secure-Systems Development

Intended Users of Technology/Tool:

Software Developers          Software Managers          Process Engineers

End Users of Application Tools

Primary Methodology Base:

Rule-based          Methodology Independent (Configurable to support any method)

Configurations:

Platform(s):	SUN SPARC, VAX/VMS, DECStation/Ultrix, IBM PS/2

OS/2 )Presentation Manager), IBM RISC 6000/AIX  (Motif)

Description/Purpose:

Technically advanced, cost-effective support for engineering system/software

development methods, application-specific requirements engineering, multi-level

security, business-specific requirements engineering, enterprise and process

modeling, development environment frameworks, standards compliance.

STSC Process Technologies Product Sheet Version 2.0b

WorkFlow Analyzer by Meta Software Corp

Product Information:

Version Number:		1.0

Release Date:  		Oct. 1993

Frequency of Updates:

Date of First Release: 	Oct. 1993

Number Sold/Released:	30-50+

Pricing:

Single User Price: 	$34,995

Site License:     	Call for info

Multi-Copy Price: 	Call for info

Upgrade Costs:	Inc. in maintenance

GSA Price:		quoted upon request

Maintenance Price: 	$5000/yr (1st yr incl)

Contact Information:

In business since: 	Dec 1985

Date research began:	1986

Address:  	125 Cambridge Park Drive

Cambridge, MA 02140

Phone Number: 	(617)576-6920

Point of Contact: 	Lou Zand

Phone Number:	(617)876-9725

Fax Number:		(617)661-2008

E-mail Address:     seltzer@metasoft.com

Electronic Bulletin Board:

Customer Support :

Process Technology Areas/Capabilities:

Definition	Process Examples	Workflow

Modeling	Business Process Redesign

Intended Application Area:

Business Process Redesign	Workflow

Intended Users of Technology/Tool:

Software Developers   	Software Managers	Process Engineers

Primary Methodology Base:

IDEF0 Process Modeling

Colored Petri Nets (transparent to user)

Configurations:

Platform(s):     Mac, X-Windows (Sun Solaris, HPUX)

Memory Requirements:     varies depending on platform

Data Import/Export Information:     ASCII, Spreadsheet

Description/Purpose:

The WorkFlow Analyzer is desktop software designed to help organizations re-engineer their business processes to achieve dramatically improved performance in costs, quality, service and speed.  The WorkFlow Analyzer toolset allows business users to develop graphical models of existing workflows, which are then automatically translated into dynamic simulation models for the identification of bottlenecks, resources utilization, and process costs.  Models can be tested under differing "what if" scenarios to show the results of proposed business changes.  WorkFlow Analyzer reporting includes both integrated spreadsheets, and easy-to-interpret graphs.

STSC Process Technologies Product Sheet Version 2.0b

tc "Appendix C -  Process Technology Product Critiques"§Appendix C -  Process Technology Product Critiques

This appendix contains a number of method/tool critiques and a template that can be used for additional process method/tool critiques.  We welcome input from readers of this report who have used process technology methods and tools and would be willing to share their  experiences with others.  All user critiques in the report are written by actual tool users.  Editing by the STSC is kept to an absolute minimum.  A critique has seven sections: (1) Reviewer's Background Information, (2) Tool Name, (3) Project Information, (4) Notable Strengths, (5) Notable Weaknesses, (6) Advice for Potential Users of Method/Tool, and (7) Vendor Comments.  The purpose of the Reviewer's Background and Project Information sections is to give a critique reader an idea of the background of the evaluator and the nature of the project using the method/tool so that the applicability of the critique to the reader's context can be judged.  Actual names of reviewers or reviewer affiliations do not appear in this report.  The Strengths, Weaknesses, and Advice for Potential Buyers sections allow free-form commentary about the tool by the evaluator.  The Vendor Comments section is allows for vendor response.

AI0-WIN Tool Critique

Reviewer's Background Info:

Position/Title:  Senior Project Engineer

Main Duties:

Tool used for software process:      yes

Years of software experience:      10

Years of experience with the tool:  3 mos

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:   10/6/93

Tool Name:  AI0-WIN

Vendor:   Knowledge Based Systems Int

Version:     2.0

Hardware platform:    Gateway 486

Operating system:     DOS 6.0

Memory used:    16 MB

Disk space used:

Enhancements:     Large Monitor

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Software maintenance process definition task

Notable Strength(s) of the Tool:

• Windows Interface

• File handling approach,

• Activity and concept pools

• Integrated with other IDEF tools

Notable Weakness(es) of the Tool:

• Poor arrow routing

• No control over activity box size

• If text is large enough to read, it's not all displayed or printed

• Size of text dependent on number of activities

• Labels can overlap eachother

• No call mechanism arrows

Advice for Potential Users of Method/Tool:

Wait for version 3 to see if the weakness listed above are corrected.

Vendor Response:

The fact that the evaluator made reference to the Pools and the integration as strengths of the tool indicates to me (vendor) that he or she had a good grasp of the workings of the tool and how best to use it.  As far as the weaknesses are concerned, many of them can currently be corrected by the user with the options available from various dialogs and user settings.  These "fine tuning" details are not currently well covered in the manual but information on how to use them can be obtained from tech support (free for one year with purchase).  All of the first five points can in some way be influenced by the user to increase the quality of the output.  The final point is one that will be addressed in Version 3.

STSC Product Critique Sheet V 2.1

Authormate Tool Critique

Reviewer's Background Info:

Position/Title:  Group Leader

Main Duties:  Developing process models

for Air Force software maintenance

Tool used for software process:    yes

Years of software experience:        10

Years of experience with the tool:       1

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:  4-April-93

Tool Name:  Authormate

Vendor:  Eclectic Solutions

Version:

Hardware platform:  Gateway 2000

Operating system:  DOS

Memory used:

Disk space used:

Enhancements:

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Project required collecting and evaluating software process models and synthesizing them into a common process model.  Also involved creating new software process models.

Notable Strength(s) of the Tool:

The tool is easy to learn and to use.  It supports diagram configuration management and the creation of IDEF kits.  It supports the maintenance and linking of multiple models.  It has a text oriented glossary that can be incorporated into a paper presentation of the model.

Notable Weakness(es) of the Tool:

The tool is very rigid in the number of boxes and arrows that can be presented in a diagram, what they look like, and how they are laid out.  It limits the number of characters that can be used to label the activities or dataflows.  The glossary is good in concept, but cumbersome in implementation.

Advice for Potential Users of Method/Tool:

Authormate is a very good tool.  I would recommend it for experienced IDEF users that are already convinced that the information presentation rules of IDEF make sense.  I would also recommend it for users that have large models and need configuration management.

Vendor Response:

Authormate limits a diagram to six boxes and the arrows on one side of an activity box

to seven, for a total of twenty-eight arrows attached to a box.  The recommended practice

for SADT/IDEF0 is a maximum of four arrows to a side and a total of nine to a box.

Although Authormate does limit the length of the title of an activity to 32 characters,

there is to limit to the length of arrow ("dataflow") labels.

STSC Product Critique Sheet V 2.1

Authormate Tool Critique

Reviewer's Background Info:

Position/Title:    SW Consultant

Main Duties:

Tool used for software process:      yes

Years of software experience:           10

Years of experience with the tool:      3

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:  11/12/93

Tool Name:  Authormate

Vendor:    Eclectic Solutions

Version:   1.0 D

Hardware platform:     PC

Operating system:      DOS

Memory used:

Disk space used:

Enhancements:

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Process Modeling of Software Development

Notable Strength(s) of the Tool:

Library, Version Control

Notable Weakness(es) of the Tool:

No networking capabilities, no support for multiple users

Advice for Potential Users of Method/Tool:

Good solid tool for IDEF.  It has no other practical use.

Eclectic has spent a good deal of time meeting our customer needs.

Vendor Response:

The recommended style of use of Authormate in a network is to distribute Kits via

a file server or e-mail.  This conforms to the SADT/IDEF0 preferred style of teamwork.  Teams of 40 analysts on three continents have worked effectively this way.

STSC Product Critique Sheet V 2.1

Authormate Tool Critique

Reviewer's Background Info:

Position/Title: President

Main Duties:  Consult on business process

engineering, SW engineering, modeling

Tool used for software process:      yes

Years of software experience:      18

Years of experience with the tool:    7

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:  12/1/93

Tool Name:  Authormate

Vendor:   Eclectic Solutions Corp

Version:

Hardware platform:  PC, VAX

Operating system:  DOS 5.0, VMS

Memory used:   DOS, - 640 K

Disk space used:

Enhancements:Herc card (PC), color VAX

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Used it on several projects over course of 7 years. Projects ranged from business process definition (e. g., manufacturing, engineering change order, collaborative work, strategic planning), software process definition, software architecture. Projects involved teams as small as 1 to as large as 40, geographically distributed.

Notable Strength(s) of the Tool:

Very high productivity - best of any IDEF-0 tool I've tried. Understands underlying IDEF semantics, including glossary, FEOs and text positioning in node structure. Excellent support for C#'s, model versions. Easy to pass diagrams among practitioners, even by email. Works well in team environment. (Newer version has checking facilities not tried.) Easy to teach in context of an IDEF-0 course with little student frustration. Handles large models well; efficient implementation does not use much disk space for diagrams.

Notable Weakness(es) of the Tool:

Rigid on box positioning -- all must be on main diagonal at fixed positions. Sometimes difficult to deal with clutter. Arrow manipulation sometimes weak.

Advice for Potential Users of Method/Tool:

Highly recommended

Vendor Response:

Release 1.2 is about 30% more efficient in its use of disk storage for models.  This

version also includes Property-Label/Value analysis and reporting facilities (including

Activity Based Costing capability)

STSC Product Critique Sheet V 2.1

Design/IDEF Tool Critique

Reviewer's Background Info:

Position/Title: Software Engineer

Main Duties:

Tool used for software process:       yes

Years of software experience:      six years

Years of experience with the tool:  one year

Last time tool used:

Currently	6 months	1 year	       >1 year

I am a software:

Manager	 Engineer  	Programmer	Novice

Date of Review:  19-Nov-93

Tool Name:  Design/IDEF

Vendor:   Meta Software Corp

Version:   2.52

Hardware platform:  Macintosh and IBM

Operating system:  MacOS and Windows

Memory used:  2.5 MB +

Disk space used:  3 MB

Enhancements:   None

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Software process modeling for several Air Force Organizations.

Notable Strength(s) of the Tool:

• Consistency checking between the pages of the model.

• Portability of models between machines (IBM and MAC) is an excellent feature especially

with multiple people working on the model.

• Overall, the tool is very robust for projects only requiring basic modeling.

Notable Weakness(es) of the Tool:

The data dictionary and glossary features are rather limited.  They are difficult to set up initially and modifying the entries can difficult.  If not done correctly, the model may become corrupted.

The Page structure display is limited.  It does not allow you to specify the exact hierarchy if you add additional pages (FEOs and Text pages, for example).

Kit support and c-number support for the configuration control of pages.

Advice for Potential Users of Method/Tool:

Keep in touch with Meta Software.  They provide excellent support for the tool.  Always feel free to make suggestions as to how they can improve the tool.

Sometimes the tool can take on a mind of its own.  It thinks it knows where an arrow (or any other object) should go better than you.  Be patient and sometimes rather than getting frustrated, you'll be better off deleting the object and starting over.

Vendor Response:

Design/IDEF v3.0, released on January 24, 1994, provides a fully-integrated glossary

feature which requires NO additional setup.  It allows capture of name, definition, alias,

and additional object information such as cost and quantity for: activities, ICOMs,

entities, attributes, and relationships.  The glossary has a comprehensive browser,

report generator, and import / export capabilities.

STSC Product Critique Sheet V 2.1

Design/IDEF Tool Critique

Reviewer's Background Info:

Position/Title: Group Leader

Main Duties:  Developing process models

for Air Force software maintenance

Tool used for software process:      yes

Years of software experience:      10

Years of experience with the tool:  1

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:  4-April-93

Tool Name:  Design/IDEF

Vendor:         Meta Software

Version:         2.5.2

Hardware platform:  Macintosh IIsi

Operating system:

Memory used:

Disk space used:

Enhancements:        large monitor

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Project required collecting and evaluating software process models and synthesizing them into a common process model.  Also involved creating new software process models.

Notable Strength(s) of the Tool:

The tool is easy to learn and to use.  It is very flexible in how IDEF diagrams can be drawn and what information can be shown.  It provides the ability to draw diagrams that violate IDEF rules.  This should be avoided, but sometimes this is the best solution.  (Or convinces you that there has to be a better solution.)

Notable Weakness(es) of the Tool:

The tool can create IDEF diagrams, but it does not actively support the IDEF development methodology, i.e., kit cycles, configuration management.  It does not actively support the call of one model from another.  It does not support the page-pair presentation format.  Its glossary is not oriented towards providing a text description of the data and is of limited use in creating a paper presentation of the model.

Advice for Potential Users of Method/Tool:

Design/IDEF is a very good tool.  I would recommend it for new users if they have adequate IDEF training that emphasizes the methodology aspects that are not incorporated into the tool.  I would also recommend it for smaller projects that do not need configuration management of diagrams or the use of embedded models.

Vendor Response:

STSC Product Critique Sheet V 2.1

Design/IDEF Tool Critique

Reviewer's Background Info:

Position/Title:  President

Main Duties:  Consult on business process

engineering, SW engineering, modeling

Tool used for software process:      yes

Years of software experience:      18

Years of experience with the tool:   1+

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:  12/1/93

Tool Name:  Design/IDEF

Vendor:   Meta Software

Version:   2.5

Hardware platform:  Mac, PC

Operating system:    Mac, Windows

Memory used:

Disk space used:

Enhancements:   large monitor (Mac)

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Used on variety of projects, most on software process definition and description. Project teams typically small, 4 or fewer. Used largely for IDEF-0. (Tool supports IDEF-1x also.)

Notable Strength(s) of the Tool:

Excellent graphics and layout capability, good arrow handling capability. Ability to draw good looking diagrams with well-placed elements is very high. Good facilities for drawing non-IDEF diagrams on FEOs (illustrative figures). Also, good link to simulation tool (not used by this reviewer).

Notable Weakness(es) of the Tool:

Version 2.5 has poor support for C# or any kind of version control. Difficult to pass diagrams among team members, and difficult to integrate single diagram into model. Generally low productivity with the tool. Tool makes underlying assumptions about the meaning of arrows (no forks and joins), and forks and joins are accomplished by overlays; this leads to difficulty on complex arrow structures, some of which cannot be drawn. Tool understands hierarchic structure for diagrams only, but doesn't recognize position of text and FEOs in model structure. Glossary facility is powerful but too complex. Tool may be frustrating to new users.

Advice for Potential Users of Method/Tool:

Graphic capability makes this tool useful. If concerned about use in large team or want re-usable diagrams or model fragments, consider other tools.

Vendor Response:

Design/IDEF has a sub-model integration feature via the 'Save Page'/'Load Page' commands.  This allows large teams to distribute pieces of a model to individual team members for development and re-integrate the sub-models back into the overall model as needed.  See description on previous Design/IDEF tool critique on integrated glossary feature.

STSC Product Critique Sheet V 2.1

iThink  Tool Critique

Reviewer's Background Info:

Position/Title:   Senior Technical Staff

Main Duties:    V+V of SW for Trident

missile.  IR+D proj of SW process model

Tool used for software process:       yes

Years of software experience:          10

Years of experience with the tool:     4

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:  12/13/93

Tool Name:    iThink

Vendor:     High Performance Systems

Version:   1.01

Hardware platform:   Macintosh

Operating system:    7.1

Memory used:         3 MB RAM

Disk space used:     903K on Hard Drive

Enhancements:     n/a

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

The goal was to build a prototype, micro-computer based, dynamic simulation of the software process.

Notable Strength(s) of the Tool:

• Graphical programming with 4 simple icons.

• Animation allows users to follow "bath-tub" flow.

Notable Weakness(es) of the Tool:

Cannot link different models - must copy and then delete/add/rename, which is very slow for

large models.

Advice for Potential Users of Method/Tool:

Seeding and running large models is cumbersome.  There are COTS front-end user interfaces which speed up the simulations once coding is complete.

Vendor Response:

STSC Product Critique Sheet V 2.1

Objectory Tool Critique

Reviewer's Background Info:

Position/Title:   Member of Technical Staff

Main Duties:    Technology Transfer

assessment, and support

Tool used for software process:      yes

Years of software experience:      10

Years of experience with the tool:      1

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:   10/4/93

Tool Name:  Objectory

Vendor:   Objective Systems

Version:    3.3.0.2

Hardware platform:  Sun Sparc IPC

Operating system:     Sun OS 4.1.2

Memory used:     48 MB RAM

Disk space used:   mem. swap~90MB ea

Enhancements:

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Current:    Customer contact service negotiation

Previous:  Multimedia information framework

Notable Strength(s) of the Tool:

Navigation:  Hypertext-like navigation and links, many ways to accomplish tasks

Notable Weakness(es) of the Tool:

Interface:  Smalltalk interface and tool interface force tedious steps to task completion

Advice for Potential Users of Method/Tool:

Maintain appropriate expectations.  Tool is well integrated with process and is improving

Vendor Response:

STSC Product Critique Sheet V 2.1

Objectory Tool Critique

Reviewer's Background Info:

Position/Title:      Senior VP

Main Duties:       Strategic Systems

development

Tool used for software process:      yes

Years of software experience:      12

Years of experience with the tool:    1

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:  9/14/93

Tool Name:  Objectory

Vendor:   Objective Systems AB, Sweden

Version:      3.3.1

Hardware platform:   Intel 486, 66 MHz

Operating system:     MS Windows 3.1

Memory used:          32 MB RAM

Disk space used:      150 MB

Enhancements:         17" SVGA Monitor

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Re-engineering and conversion of a 3090 non-GUI COBOL application into a distributed C++ client/server based environment on "top" of NCR's CooperationR.  Windows 3.1 client user interface with server running on UNIX System V, Release 4

Notable Strength(s) of the Tool:

Its unparalleled depth and adherence to the only commercially (large scale) proven methodology for object-oriented software engineering as developed by Dr. Ivar Jacobson and the Ericsson Group since 1967!  This significant industrial software development effort was not made public until the late 1980's due to the advantage it gave the Ericsson Group in its unique and very competitive global market place.  The tool is perfect not only for software engineering, but perhaps more importantly for the execution of business and/or process engineering prior to any software development.

Notable Weakness(es) of the Tool:

The postscript based text and format editor for document handling are somewhat cumbersome to work with when used with Word for Windows and similar window-based word processors.  However, a large volume of text can be "imported" from a more modern word processor by the CUT & PASTE method and placed into the appropriate ObjectoryR document.  It is our understanding that this might soon be a problem of the past.

Advice for Potential Users of Method/Tool:

The OOSE team should attend classes in object-oriented analysis and design and an OO language in preparation for any OOSE work including ObjectoryR.  Excellent classes are offered by Semaphore and AT&T/NCR.  Furthermore, it helps to read Dr. Jacobean's book "Object Oriented Software Engineering - A Use Case Driven Approach"  [Addison-Wesley ISBN 0-201-54435-0, May 1992]

Vendor Response:

STSC Product Critique Sheet V 2.1

Process Weaver Tool Critique

Reviewer's Background Info:

Position/Title:  Senior Systems Eng.

Main Duties:   Software Eng Environments,

Program Management Processes

Tool used for software process:      no

Years of software experience:

Years of experience with the tool:

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager     Systems Engineer     Programmer       Novice

Date of Review:  12/7/93

Tool Name:  Process Weaver

Vendor:   Cap Gemeni

Version:   1.2

Hardware platform:  RISC 5/6000

Operating system:   AIX

Memory used:

Disk space used:

Enhancements:

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Currently integrating a project management Tool with an RDBMS, publishing tool, and a

metrics tool to create a program management system.  The program management

process is enacted via Process Weaver

Notable Strength(s) of the Tool:

• Task description builder, no need for a separate GUI builder

• Interface to a project management tool

• Handles complex processes well

• Intuitive graphical interface for defining processes

• Functionally rich (covers almost any possible branch rule)

Notable Weakness(es) of the Tool:

• Data integration (fixed in current Beta version)

• No integrated data base (stores workflow info in unix files)  (PCTE interface imminent)

• Poor user documentation, but has been improving

Advice for Potential Users of Method/Tool:

• Take classes offered by Cap Gemeni

Vendor Response:

• Version 2.0 has file access and PCTE Integration

• Models will be stored in PCTE

• Version 2.0 will have Hypertext online help and product will ship with tutorials

• Consulting Support (mentoring)

STSC Product Critique Sheet V 2.1

ProSim Tool Critique

Reviewer's Background Info:

Position/Title:Indust. Proc. Impr Prg Mngr

Main Duties: Consultant for those with

needs in process analysis+improvement,

Tool used for software process:      yes

Years of software experience:      5

Years of experience with the tool:     1

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:  10/15/93

Tool Name:ProSim (formerly AI3)

Vendor:   Knowledge Based Systems, Inc

Version:   1.0

Hardware platform:    386 PC

Operating system:     WIndows 3.1

Memory used:    2 MB

Disk space used:  2 MB for software pkg

Enhancements:   none required

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Have trained over 250 personnel in the use of the tool.  Have some 2 dozen known process definition efforts ongoing.  Most of these projects are in the diagramming phase only, but a few are going to simulation

Notable Strength(s) of the Tool:

1-  Database Driven, makes for consistent propagation of changes across all scenarios and decompositions.  Also enables export and import to and from tool.  Makes for true enter once - use many data manipulation.

2-  Does drafting chores.  Has automated router which combines with graphical elements to draw the diagram for the user, eliminating all time usually spent on picking a drawing tool, sizing the element, placing the element, and then connecting the elements.  Makes for very quick diagramming.

3-  Rigid implementation of IDEF3.  Will only do IDEF3 process descriptions.   Educates the user in IDEF3, standardizes process description products for easy transfer of meaning from user to all readers.

4-  Generates WITNESS simulation model.  Uses adequate process description (no magic bullet for getting by the data issues of simulation) to generate simulation model which accurately contains the elements and logic of the process.  Easier (user friendly, quicker drawing) to use ProSim, and then go to WITNESS than to do initial engineering assessment and then start WITNESS model from scratch.  This is more true for the WITNESS novice, and less true for the WITNESS expert.

5-  Works on three levels of enhancing understanding of process.  The first level is the diagramming phase, which is very easy for process analysis novices to learn in as little as two hours of training.  The second level is the IDEF3 elaboration, which is more sophisticated, but not impossible for novices to pick up, especially the fundamentals.  This is where the true guts of the nature of the process begins to surface for evaluation.  The third level is the simulation, which is the least likely for novices to use.  However, it does mean the same tool can be used to transition from novice to simulation expert without changing software.

Advice for Potential Users of Method/Tool:

We have had one person who thought that all 100 units of behavior (UOB) of a process needed to be displayed on one screen.  It took about 7 minutes for a 50 megahertz 486 to run the routing algorithm and display the diagram.  Others who used scenarios and decompositions on  processes having this many or more UOBs have had no problem with slow display times.  (do 100 UOB process diagrams really communicate any more than WOW?!)  Vendor support on this issue has been excellent, and future versions will address this characteristic. In addition, while the simulation model is accurate, it is not what we have called "visually intuitive".  The WITNESS software can draw a shop floor layout which looks just like the shop floor and enhances the viewer's understanding.  This must still be done in WITNESS.  I don't know if it is fair to say it is a weakness, it is not a "notable" weakness.  But it does provide perspective on where ProSim ends and WITNESS begins.

Advice for Potential Buyers of this Tool:

I find this tool unique in the current tool selection available.  I am thoroughly persuaded of its superlative performance and process analysis power.  IDEF3 beats IDEF0 hands down for comprehensive process analysis, and the simulation link to WITNESS enhances that power to another level.

Vendor Response:

We (the vendor) feel that the evaluator made a fair and accurate evaluation of the product.  The large number of UOBs displayed-on-one-screen problem will be dealt with in the next version for the rare occasion that one wants to display a model of their processes in this manner.

STSC Product Critique Sheet V 2.1

RDD-100 Tool Critique

Reviewer's Background Info:

Position/Title:  Sr. Member Tech Staff.

Main Duties:  System Engineering and

Collaborative Environments Development

Tool used for software process:    yes

Years of software experience:  13

Years of experience with the tool:  1.5

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:  04/13/93

Tool Name: RDD-100

Vendor:      Ascent Logic Corp.

Version:     3.1

Hardware platform:  Sun , DEC, Mac

Operating system:  Sun O/S, Ultrix, Mac

Memory used:  16Mb min., 24Mb sugg.

Disk space used:  40 Mb min.

Enhancements:  19” monitor

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

We’re integrating RDD into an environment framework as part of an integrated product development system.  We are also using RDD on a Sun SPARC IPC to design this system.

Notable Strength(s) of the Tool:

Excellent customizable system engineering design database tool, based on the “Mack Alford system engineering methodology”.  Supports full (semantic) traceability for system requirements decomposition to functional decomposition (and simulation) to component decompositions in an E-R database.  Has built in design consistency checking. Designed for team collaboration.  Has substantial automated Mil-Std 499B and 2167A  document generation capabilities as well as interfaces to other frameworks and tools.

Notable Weakness(es) of the Tool:

Users need substantial training to be effective, particularly for simulation.  Graphical interface is a mixed bag.  Not Xwindows compatible (due 3/93).  Current packages, basic and simulation, are quite expensive, however vendor is unbundling the capabilities into smaller, less expensive packages.

Advice for Potential Users of Method/Tool:

Take the class first.  The RDD-100 system is VERY comprehensive; a “Cadillac” that’s not for the faint-hearted.

Vendor Response:

STSC Product Critique Sheet V 2.1

Statemate Tool Critique

Reviewer's Background Info:

Position/Title:   Staff Engineer

Main Duties:    Software Development

Tool used for software process:      yes

Years of software experience:      20+

Years of experience with the tool:

Last time tool used: (see proj info)

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:  11/12/93

Tool Name:  Statemate

Vendor:     I-Logix

Version:    2.5

Hardware platform:     VAX

Operating system:    VMS

Memory used:

Disk space used:

Enhancements:

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

I am currently using "the Statemate language" supported with Meta Design 2.0 on a Cobra

helicopter cockpit integration project.  I used version 2.5 on a project several years ago.  I

have seen a demo of the new version which has a much improved user interface.

Notable Strength(s) of the Tool:

"The Language" is a very useful medium for helping to understand and describe the behavior of a system.  The tool augments the analysis process by facilitating the 'simulation' of a systems behavioral logic.

Notable Weakness(es) of the Tool:

Both the language and the tool require a high training cost (but you get what you pay for).

Advice for Potential Users of Method/Tool:

- Understand that this is a systems analysis tool.

- Underhand what you want the tool to help you with.  It won't lead you.

- Understand, in depth, The Statemate Language.  The tool won't teach it to you or show you

how to use it.

Vendor Response:

The user's observation regarding the use of Statemate is accurate in that the tool doesn't lead the user in any one path.  It facilitates and augments the system engineering function through better communication, analysis, and earlier detection of potential problems than with traditional approaches.  Statemate also provides links to downstream tools, eliminating the past need to "start from scratch" as a design progresses.   The user's observation regarding training is accurate, but not complete.  Statemate is both a language and a different approach to systems design.  Training minimizes the learning curve any engineer must go through when adopting new tools.  There is a cost associated with training, both in time and dollars.  Although it is hidden, there is a cost of not having training.  That cost is in terms of lost productivity, of user discouragement, and of software becoming "shelfware".  It is not in the best interest of both the customer, and us to let these things take place, hence the emphasis on training.

STSC Product Critique Sheet V 2.1

ProEditor/VPML Tool Critique

Reviewer's Background Info:

Position/Title: Software Engineer

Main Duties:  Project lead of documenting

software development process

Tool used for software process:      yes

Years of software experience:      6

Years of experience with the tool:      0.5

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:  September 7, 1993

Tool Name:       VPML

Vendor:       Internat'l SW Systems Inc.

Version:   1.3 (Alpha)

Hardware platform:  Sun Workstation

Operating system:   UNIX

Memory used:   24 MB

Disk space used:  50 MB  (120 MB Swap)

Enhancements:    Sparc 10, Large Monitor

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Defining/Capturing software development process for F-16 imbedded computer systems.

Notable Strength(s) of the Tool:

• Graphical representation of process

• Ease of use/training minimal

• Diagrams are easy to understand

Notable Weakness(es) of the Tool:

• Large processes require too long to load into memory

• No configuration management incorporated into tool.

Advice for Potential Users of Method/Tool:

Vendor Response:

In version 2.2, we significantly improved both timing and sizing performance.

Since process models are stored as Unix files, sccs or rcs can be used to control versions

of the process models.   In addition, we have added the suggestion to incorporate

configuration management into ProEDITOR to our list of user requests and will consider

it for inclusion in future releases.

STSC Product Critique Sheet V 2.1

Sample Product Critique Sheet

Reviewer's Name:

Company Name:

Position/Title:

Main Duties:

Tool used for software process:      yes/no

Years of software experience:

Years of experience with the tool:

Last time tool used:

Currently	6 months	1 year	>1 year

I am a software:

Manager	Engineer	Programmer	Novice

Date of Review:

Tool Name:

Vendor:

Version:

Hardware platform:

Operating system:

Memory used:

Disk space used:

Enhancements:

(accelerator, large monitor, graphics card, etc.)

Overall impression of the tool?

Excellent	Good	Fair	Poor

Quality of vendor support?

Excellent	Good	Fair	Poor	Unknown

Project Information:

Notable Strength(s) of the Tool:

Notable Weakness(es) of the Tool:

Advice for Potential Users of Method/Tool:

Vendor Response:

STSC Product Critique Sheet Version 2.0e

tc "Appendix E - IDEF Training Sheets"§Appendix E - IDEF Training Sheets

Many of the STSC's customers were using, or were interested in using, the IDEF technology.  We thought it would be extremely important to aid them in finding proper training for the different levels of IDEF usage that exist.  All too often tools are bought and never used because proper training was never acquired.  We had a goal of assisting organizations learn about process tools, but additionally give them information on where to look for training to get them up and working in the targeted methodology area.

To ensure correctness/timeliness of the following information, be sure and contact the company of interest.  In addition, the STSC can be contacted for new/updated IDEF training information by calling their customer service department at (801)777-7703 or DSN 458-7703, fax to  (801)777-8069 or DSN 458-8069, or email to godfreys@wpo.hill.af.mil.

IDEF Training at D. Appleton Company, Inc.

Company:  	D. Appleton Company, Inc.

Phone:  (214)869-1066

Fax:	(214)869-1099

Address:

222 W. Las Colinas Blvd., Suite 1141

Irving, TX  75039

Contact:

Mark Shipman

SW Product Manager

Phil Shinar

Training Director

(310)374-3939 (CA Office)

Number of IDEF Courses Offered:   Two (2)

COURSE 1 of 2: Modeling for Managers Seminar       [Intro./Readers Course]

Abstract:

D. Appleton recommends this seminar in order to minimize the effects of the "baptism of fire" that middle managers experience upon being assigned information planning and implementation projects.  It will give an understanding of the rationale behind using process and data models in  the design and implementation of information systems as well as a working knowledge of the tools and methods necessary for effective modeling.  The student will be able to read high-level IDEF models at the end of the course.  This seminar is also appropriate for business practitioners who will review and validate the models

What you will learn:

•  How to read  high-level IDEF models

•  Working knowledge of the IDEF tools and methods.

# Days: One (1)

# Students:  maximum of 15 students

Cost (minus expenses):  	$295 per student

$3000 as an on-site course

Course Location: Scheduled at various sites in U.S.  Courses also available on site.

COURSE 2 of 2:   Process & Data Modeling Workshop Using IDEF Techniques    [Authors Course]

Abstract:

This workshop demystifies the modeling process.  The workshop, driven by case studies that use IDEF Modeling Techniques, enables you to design and implement more efficient and effective automated services for your organization.  This workshop provides a non-threatening environment where end-users and data processing professionals together build and critique process and data models, stressing the discovery and documentation of business rules implicit in the organization.  Through a series of lectures, readings, exercises, and team case studies, the student will gain a working knowledge of process modeling (IDEF0) and data modeling (IDEF1X)

What you will learn:

•  How to build + critique process + data models

•  Working knowledge of IDEF0 and IDEF1X

# Days: Four (4)

# Students:  maximum of 15 students

Cost (minus expenses):	$995 per student as public course

$10,500 as on on-site course

Course Location: Scheduled at various sites in U.S.  Courses also available on site.

NOTES:

Tools/Platforms Used:

D. Appleton tries to keep their courses tool independent (they are sensitive to getting caught up in computer/tool logistics rather than concentrating on the course), but when tools come into play, they use their IDEF Leverage for IDEF0 and IDEF1x, and their ModelPro  tool for IDEF1x work.  Computers must be supplied by client firm.

Materials Used/Kept by Students:

Book following along with the modules studied in the class.

IDEF1/IDEF1X Training Offered?:

IDEF1X training is incorporated into the 4 day course, but IDEF1 is not addressed by D. Appleton.

Many Exercises/Hands On?:

Yes.  The course involves a series of exercises and team case studies.

Can Course be Tailored to SW Process Modeling?:

While D. Appleton has a lot of experience in a wide range of modeling areas, their target market is primarily  data modeling applications.

Misc.::

• D. Appleton also offers several other workshops & seminars such as Advanced Data Modeling, Enterprise

Integration, Process and Data Modeling Techniques, Process Improvement Methodology, and many more.

• Call 1(800)322-6614 for more information on D. Appleton IDEF training  options.

IDEF Training at Eclectic Solutions

Company:    Eclectic Solutions

Phone:   (619) 454-5781

Fax:	  (619) 459-3025

Address:

5580 La Jolla Boulevard

Suite 130

La Jolla, CA  92037-7692

Contact:

Pat Duran or Al Irvine

Number of IDEF Courses Offered:   Three (3)

COURSE 1 of 3:    IDEF0 Overview            [ Intro. Course]

Abstract:

This course is a management overview

What you will learn:

•  General IDEF0 description

•  IDEF0 Applicability

•  IDEF0 Resource requirements

•  Comparison to other approaches

# Days: one (1)

# Students:  No limit

Cost (minus expenses):  $2500

Course Location: Usually customer site

COURSE 2 of 3:    3-Day Intensive IDEF0  Authoring Course     [Author's Course]

Abstract:

Full IDEF0 Author's course

What you will learn:

•  Interviewing

•  Model Authoring

•  Commenting + Responding

•  Project Control

•  Use of Automated IDEF0 Modeling Environment

•  Two Case Studies

# Days: three (3)

# Students:  Maximum of eight (8) students

Cost (minus expenses):  1-4 students  $5000;   5-8 students $7000

Course Location: Usually customer site

COURSE 3 of 3:    5-Day IDEF0 Authoring Course             [Author's Course]

Abstract:  Full IDEF0 Author's course

What you will learn:

•  Interviewing

•  Model Authoring

•  Commenting + Responding

•  Project Control

•  Use of Automated IDEF0 Modeling Environment

•  Two Case Studies

# Days: five (5)

# Students:  Maximum of twenty (20) students

Cost (minus expenses):  1-12 students $10,000 ;       13-20 students $15,000

Course Location: Usually customer site

NOTES:

Tools/Platforms Used:

Although it is not required, Eclectic Solutions recommends that their IDEF0 modeling product, Authormate_, be used in both the 3-Day and the 5-Day courses.   Since Eclectic Solutions are not as familiar with other tools, they don't feel as comfortable with knowing the impact it will have on the class, how robust it is, etc.

Materials Used/Kept by Students:

Students use and keep a book of slides as well as a text on SADT, both provided by the instructor.

IDEF1/IDEF1X Training Offered?:

No... Eclectic Solutions concentrates on IDEF0 training/products only.

Many Exercises/Hands On?:

Yes... There are hands on exercises in class, as well as evening assignments.

Can Course be Tailored to SW Process Modeling?:

Yes.  The second of the two case studies examined in the course actually looks at a SW process model.

Misc.:

• The client provides the computers.  Eclectic will provide their software for use in the course.

• Other courses  offered by Eclectic:  Structured Analysis, Structured Design, and Information Modeling

IDEF Training at Knowledge Based Systems, Inc.

Company: Knowledge Based Systems, Inc..

Phone:  (409)696-7979

Fax:	(409)696-7277

BBS:	(409)696-7055

Address:

2746 Longmire

College Station, TX

77845-5424

Contact:

David N. Rice : Project Manager - IDEF Tools

Mike Graul : Training Staff

Number of IDEF Courses Offered:   Five (5)

COURSE 1 of 5:    Complete IDEF0 Course        [Author's Course]

Abstract:

KBSI provides you with the competence needed to effectively apply IDEF0 modeling methodologies.

What you will learn:

•  A detailed  explanation of the IDEF0 methodology/framework

•  Usage through extensive practical exercises involving hands-on experience IDEF0

•  An introduction to computer-aided modeling through the use of their IDEF0 tool, AI0.

# Days: flexible

# Students:  Prices quoted for ten students.  For each additional student, add $200.

Cost (minus expenses):  $5500

Course Location: Customers site

COURSE 2 of 5:    Complete AI0 Course       [KBSI's Tool Course ]

Abstract:

KBSI has acquired extensive knowledge of the modeling process through the development of their automated IDEF tools.  These tools eliminate the tedious aspects of model development and allow the modeler to concentrate on arranging efficiently structured systems.  By incorporating these automated tools into the training course, the student can concentrate on the methodology, due to the elimination of laborious hand drawing of models.

What you will learn:

•  Using KBSI's AI0 tool to develop IDEF0  models.

# Days: flexible

# Students:  Prices quoted for ten students.  For each additional student, add $200.

Cost (minus expenses):  $5500

Course Location: Customers site

COURSE 3 of 5:    Complete IDEF0 and AI0 Course [Authoring Course ]

Abstract:

KBSI integrates their AI0 tool into their IDEF0 author's course for this comprehensive authoring  course.

What you will learn:

•  A detailed  explanation of the IDEF0 methodology/framework

•  Usage through extensive practical exercises involving hands-on experience IDEF0

•  An in depth seminar in computer-aided modeling through the use of KBSI's IDEF0 tool, AI0.

# Days: flexible

# Students:  Prices quoted for ten students.  For each additional student, add $200.

Cost (minus expenses):  $5900

Course Location: Customers site

COURSE 4 of 5:    Complete IDEF1 Course       [Authoring Course ]

# Days: flexible

# Students:  Prices quoted for ten students.  For each additional student, add $200.

Cost (minus expenses):  $5900

Course Location: Customers site

COURSE 5 of 5:    Complete IDEF1X Course       [Authoring Course ]

# Days: flexible

# Students:  Prices quoted for ten students.  For each additional student, add $200.

Cost (minus expenses):  $5900

Course Location: Customers site

NOTES:

Tools/Platforms Used:

KBSI offers generic IDEF training as well as  courses specific to their IDEF tools and also combinations of the two.

Materials Used/Kept by Students:

They  provide all the materials needed for the course.  Students can keep most of the materials used.

IDEF1/IDEF1X Training Offered?:

Yes....Both courses are priced at $5900  (see above)

Many Exercises/Hands On?:

They advertise "Extensive practical exercises" in the client's chosen application area.

Can Course be Tailored to SW Process Modeling?:

KBSI is very flexible and has a large library of models already built.  While KBSI deals with mostly manufacturing clients, SW Process modeling can be handled by them too.

Misc.:

• KBSI also has courses in IDEF3, IDEF4, IDEF5 (each are $7000), and Systems Development Frameworks.

• If  KBSI's standard courses aren't exactly what you need  they  say " We send in our experts early to gain an

understanding of your company and of your special problem types.  From this information, we develop a

training course that takes into account your company's operational procedures and addresses the problems

to be solved."

• They seem very willing to work with the customer to tailor their courses to specific needs.

IDEF Training at Lipka Software Engineering, Inc.

Company:  Lipka Software Engineering, Inc.

Phone:  (617)965-0911

Fax:	(617)965-0911      (same number)

Address:

29 Chatham Road

Newton, MA  02161

Contact:

Stephen E. Lipka, Ph.D.

President

Number of IDEF Courses Offered:   Four (4)

COURSE 1 of 4    IDEF0 Readers Course  (Course #: GEN-1)           [Reader's Course]

Abstract:

This course, lasting approximately one hour, introduces the viewer to IDEF0 activity models and enables them to read diagrams and models for content.  It is intended for those who with to understand model content, follow modeling progress, or participate as interested parties in projects using IDEF0 activity models.  The course is intended for general audiences.

What you will learn:

• An introduction to IDEF0 activity models.

• To read IDEF0 models for content

# Days:  One (1) Hour

# Students:  flexible

Cost (minus expenses):  negotiated

Course Location: Customers Site.

COURSE 2 of 4    IDEF0 Commenters Course (Course #: GEN-2)      [Commenter's Course]

Abstract:

This course, lasting approximately one hour, builds on the Readers' Course.  It shows the viewer how to read the models critically and comment in a structured way to help the model's authors improve it.  The course is intended for general audiences.

What you will learn:

• An introduction to IDEF0 activity models.

• To read IDEF0 models for content

• To read IDEF0 models with a critical eye, and comment intelligently

# Days:  One (1) Hour

# Students:  flexible

Cost (minus expenses):  negotiated

Course Location: Customers Site.

COURSE 3 of 4    IDEF0 Modeling Course (Course #: MOD-1)      [Author's Course]

Abstract:

This five day course covers principles of activity modeling using IDEF0, particularly for use in enterprise modeling and system definition.  The course covers model objectives, syntax, semantics, model quality, dynamics, mechanisms, review practices, knowledge acquisition, and presentation of models.  The fifth day includes time for a practical exercise related to the students' business.  The course is highly technical and is oriented toward analysts and designers.

What you will learn:

• How to build IDEF0 models

• How to gather and extract information needed for your models

• Review practices

• How to apply what you've learned to a real life situation within your organization

# Days:  Five (5)

# Students:  maximum of 10 students (will discuss having more students)

Cost (minus expenses):  $10,000

Course Location: Customers Site.

COURSE 4 of 4    Advanced IDEF0 Modeling Course (Course #: MOD-2)      [Advanced Author's Course]

Abstract:

This four day course is oriented toward analysts and designers who have attended an introductory course with one to two days of IDEF0.  As with the five day course, this workshop covers principles of activity modeling using IDEF0, particularly for use in enterprise modeling and system definition.  The emphasis, however, is on the advanced concepts, presented through a larger number of exercises.  The course covers model objectives (particularly for As-Is and To-Be business process models), model quality, dynamics, mechanisms (including their use for showing organization and information systems), multiple model structures, review practices, knowledge acquisition, presentation of models, and use of the models for gathering costing and economic information.  The fourth day includes time for a practical exercise related to the students' problem.  Participants are expected to have knowledge of IDEF0 diagram and model syntax and semantics and are expected to have developed IDEF0 models.  Refresher material will be presented as needed.

What you will learn:

• How to build IDEF0 models

• How to gather and extract information needed for your models

• Review practices

• How to apply what you've learned to a real life situation within your organization

• More in depth work on examples

# Days:  Four (4)

# Students:  maximum of 10 students (will discuss having more students)

Cost (minus expenses):  $8,000

Course Location: Customers Site.

NOTES:

Tools/Platforms Used:

Flexible.  If working with a modeling tool is desired (this is not a requirement), Eclectic Solution's Authormate and Meta's Design/IDEF tools have been used in Mr. Lipka's courses.

Materials Used/Kept by Students:

A IDEF course book (containing all slides used in course), a set of hand-out exercises (copywritten), and workforms (public domain) to be used.

IDEF1/IDEF1X Training Offered?:

Mr. Lipka offers a course in information modeling which covers IDEF1 and IDEF1X.

Many Exercises/Hands On?:

There are exercises given, which will be reviewed that night by the instructor, to allow feedback the next day.  Also, time is allotted in the last day to work specifically on the customer's chosen application.

Can Course be Tailored to SW Process Modeling?:

Yes.  Mr. Lipka has experience in SW process modeling using IDEF and will be happy to include this information as well as show SW process examples.  However, preliminary IDEF techniques will be taught first.  This prevents people from getting bogged down in discussions about the content of the models as opposed to the techniques used to build them.

Misc.:

• References can be provided.

• Customer supplies the computer and software resources for the course when tools are used.

• Often hired as a consultant for SofTech

• Was team leader for SofTech on the first IDEF CASE tool.

• Developer of the public domain courses on reading and commenting, available through STSC.

IDEF Training at Meta Software Corporation

Company:  Meta Software Corporation

Phone:		(617)576-6920

(800)227-4106

Fax:		(617)576-0519

Address:

125 Cambridge Park Drive

Cambridge, MA  02140

Contact:

Bob Seltzer / Lou Zand

Number of IDEF Courses Offered:   Two (2)

COURSE 1 of 2:    Business Process Modeling            [Intro. Course]

Abstract:

The purpose of this course is to produce capable modelers who can use Design/IDEF to create effective business process models.

What you will learn:

•  How to use the basic  IDEF constructs.

•  How to use the Design/IDEF tool

•  How to set-up IDEF models for activity-based costing (ABC)

•  How to set-up IDEF models for simulation.

•  How to connect your IDEF models to other tools.

# Days: One (1)

Cost (minus expenses):  $500.  15% discount for two or more people from the same company.

Course Location: Course at Meta Software.  Custom courses at the customer site are available.

COURSE 2 of 2:    Business Process Analysis  [Readers/Users Course]

Abstract:

The purpose of this course is to provide business modelers with a foundation to analyze specific business problems, translate them into effective behavioral models, and deliver measurable results.

What you will learn:

•  How to translate IDEF models into CPN simulation models.

•  How to use the Design/CPN simulation tool.

•  How to perform cost/benefits analysis

•  How to produce easy-to-understand graphic presentations.

# Days: Two (2)

Cost (minus expenses):  $1000    15% discount for two or more people from the same company.

Course Location: Course at Meta Software.  Custom courses at the customer site are available.

NOTES:

Tools/Platforms Used:

Meta's tools, Design/IDEF and Design/CPN, are used in their courses.

IDEF1/IDEF1X Training Offered?:

No.  IDEF1 + IDEF1X are not covered.

Many Exercises/Hands On?:

Yes.  Using Meta Software's Design/IDEF tool is a major emphasis of the class.

Can Course be Tailored to SW Process Modeling?:

No.  Meta Software works primarily in the business process re-design arena.

Misc.:

•  The two courses together cost $1250.

•  Meta Software offers consulting services, but they are not really in the IDEF training business, even though they offer some courses at their site from time to time.  They often recommend other training options to their customers.

IDEF Training at New England Business Consultants

Company:  New Eng. Business Consultants

Phone:  (508)794-0375

Fax:	(508)685-3719

Address:

61 Elm Street,  Suite 100

Methuen, MA   01844

Contact:

Daniel E. Thornhill

Number of IDEF Courses Offered:   Twelve (12)

COURSE 1 of 12:    IDEF0 Executive Overview    # NIE-10G [Overview Course]

Abstract:

In this high-level course, one day is spent familiarizing students with IDEF0 concepts as

well as why IDEF0 is important.  Not a model developer's course.

What you will learn:

•  Why IDEF0 is important

•  How IDEF0 can be best used

•  High-level view of IDEF0 concepts

# Days: One (1)

Cost (minus expenses):  $2000

Course Location: At customer provided facility.

COURSE 2 of 12:     IDEF0 Management Course     #NIM-01D    [Overview Course]

Cost (minus expenses):  $2500

Course Location: At customer provided facility.

COURSE 3 of 12:  IDEF0 Management Course & Workshop   #NIS-11E   [Overview & Workshop Course]

What you will learn:

•  Basic management overview course with a workshop in which the customer's specific area of interest in examined.

Cost (minus expenses):  $5000

Course Location: At customer provided facility.

COURSE 4 of 12:     IDEF0 Reader Course     #NIR-02J           [Reader's Course]

What you will learn:

•  Full IDEF0 Model Syntax

•  How to correctly read and understand IDEF0 models

Cost (minus expenses):  $500

Course Location: At customer provided facility.

COURSE 5 of 12:    IDEF0 Reviewer Course     #NIC-03B    [Reader/Reviewer Course]

What you will learn:

•  How to correctly read and understand IDEF0 models

•  Make corrections and comments on IDEF0 models

Cost (minus expenses):  $1000

Course Location: At customer provided facility.

COURSE 6 of 12:    IDEF0 Author Course     #NIA-04M        [Author's Course]

What you will learn:

•  Management techniques for projects utilizing IDEF0.

# Students:  Max. of 20 students

Cost (minus expenses):  $8,000 up to 10 students,  $750 for each additional, up to 20 students

Course Location: At customer provided facility.

COURSE 7 of 2: IDEF0 Author Course and Modeling Workshop  #NII-05P [Author's Course & Workshop]

Abstract:

Lectures and an integrated example present IDEF0 fundamentals.  A "standard" workshop provides group practice after presentation of each concept.  A "tailored"  workshop provides group and individual application of the concepts to develop an IDEF0 model of some aspect of the customer's project or company,  Instructors give  careful attention to individual work.  Outside reading and homework assignments provide reinforcement and practice.

What you will learn:

•  IDEF0 background + syntax

•  Modeling procedures and information gathering

•  Developing complete models, text, glossaries, and figures

•  Model review, validation, and revision

# Days: Five (5)

# Students:  Max. of 20 students

Cost (minus expenses):  $12,000  up to 10 students.   $1,000 for each additional, up to 20 students.

Course Location: At customer provided facility

COURSE 8 of 12:   IDEF0 Project Workshop   #NIP-06C         [Project Workshop Course]

Cost (minus expenses):  $5000

Course Location: At customer provided facility.

COURSE 9 of 12:   Advanced IDEF0 Author Course      #NIF-07A    [Author's Course]

What you will learn:

•  Advanced techniques and concepts of IDEF0 modeling

Cost (minus expenses):  $6000

Course Location: At customer provided facility.

COURSE 10 of 12:   Advanced IDEF0 Author Course and Project Workshop   #NIG-08L

[Author's Course and Project Workshop]

What you will learn:

•  Advanced techniques and concepts of IDEF modeling

•  Your specific case  is examined and discussed.

Cost (minus expenses):  $10,000

Course Location: At customer provided facility.

COURSE 11 of 12:     IDEF Project Librarian Course   #NIL-09S    [Librarian Course]

What you will learn:

•  Issues important to those who will be keeping track of models produced/shared will be discussed

Cost (minus expenses):  $500

Course Location: At customer provided facility.

COURSE 12 of 12:     IDEF1X Data Modeling Course  #NID-12A    [Author's Course]

What you will learn:

•  Syntax, concepts, and issues of IDEF1X modeling

Cost (minus expenses):  $8000

Course Location: At customer provided facility.

NOTES:

Tools/Platforms Used:

Do not  concentrate on tools as much as the methodology so that students will not get caught in issues of tool operation.  However,  NEBC will incorporate customer tools into courses if that is desired.

Materials Used/Kept by Students:

• Course workbook with copies of all slides

• SADT textbook by Marca and McGowan

• Custom IDEF  mouse pad

• IDEF template, diagram pad, forms, colored pens, and other supplies

• Sample models and other handouts

IDEF1/IDEF1X Training Offered?:

One course on IDEF1X modeling is offered.  However, most of their courses are designed around IDEF0.

Many Exercises/Hands On?:

Yes.  Exercises and  Homework are an integral part of the class.

Can Course be Tailored to SW Process Modeling?:

Their courses are built around business process modeling, rather than SW Process Modeling

MISC:

• Dan was involved in the development of training materials for SofTech and is often hired as a consultant by SofTech.

• Willing to use customer-focused case examples in class for discussion/review.

IDEF Training at Productivity Solutions

Company:  Productivity Solutions (PS)

Phone:  (401) 847-8875

Fax:

Address:

30 East Street

Newport, RI      02840

Contact:

John E. Stockenberg,  Ph. D.

Chief Technical Officer

Number of Courses Offered:   One (1)

COURSE 1 of 1:    IDEF0 Activity Modeling Techniques           [Author's Course]

Abstract:

This course will include several group and individual exercises designed to demonstrate the applicability of IDEF0 techniques to command and control and other embedded real-time system analysis tasks.

What you will learn:

•  Intro. to Modeling  and IDEF diagram syntax

•  Model decomposition and hierarchy

•  Creating a Model:  context, viewpoint, purpose, interviewing  tips, conventions for readability

•  Building a Data Dictionary

•  How to Review/Comment on a model

•  The Author / Reader Cycle

•  Using Models for Project Plans

•  Mapping to DOD-STD-2167A, MIL-STD-490A, SSS, PIDS, CIDS, SRS

# Days: Three (3)

# Students:  Up to 20 students

Cost (minus expenses):  $1500

Course Location: At customer's site

NOTES:

Tools/Platforms Used:

This course has been taught with students using paper and pencil.   John Stockenberg is familiar with

Meta Software's Design/IDEF and could include some tool use in the course, but doesn't have past experience doing this.

Materials Used/Kept by Students:

Students use and keep a 150-200 page spiral-bound book of viewgraphs.

IDEF1/IDEF1X Training Offered?:      No.

Many Exercises/Hands On?:   Paper/pencil examples are worked through, but this course hasn't been taught using  computers and software tools.

Can Course be Tailored to SW Process Modeling?:

No.  He has worked with people involved in embedded systems and has examples of how to use IDEF as a planning tool, but doesn't have experience  teaching SW process modeling.

Misc.:

• Other courses are available to supplement the IDEF0 course.  Available topics include behavior modeling,

SCR/A-7E/NRL/Parnas design and specification techniques, and Petri net modeling.

• As stated above, John has not taught this course using actual computers/IDEF tools.  The customer would

have to  provide the resources (computers/software) for this to take place.

IDEF Training at Wizdom Systems

Company:  Wizdom Systems, Inc.

Phone:  (708)357-3000

Fax:	(708)357-3059

Address:

1260 Iroquois Avenue

Naperville, IL  60563

Contact:

Kristy Hanna

or

Tom Newman

Number of IDEF Courses Offered:   Twelve (12)

COURSE 1 of  4:    IDEF0 with IDEFine-0   [Reader's/User's Course]

What you will learn:

•  The IDEF0 methodology using Wizdom Systems' tools.

# Days: Four (4)

# Students:  Maximum of 10

Cost (minus expenses): 	$12,605 w/ one copy of the IDEFine Family tool suite installed/delivered.

$25,105 w/ four copies of the IDEFine Family tool suite installed/delivered.

Course Location:  This course is offered only to on-site client firm only.

COURSE 2 of  4:    IDEF1x with IDEFine-1x                [Reader's/User's Course]

What you will learn:

•  The IDEF1x methodology using Wizdom Systems' tools.

# Days: Four (4)

# Students:  Maximum of 10

Cost (minus expenses): 	$11,105 w/ one copy of the IDEFine Family tool suite installed/delivered.

$17,055 w/ four copies of the IDEFine Family tool suite installed/delivered.

Course Location:   This course is offered only to on-site client firm only.

COURSE 3 of  4:    Advanced IDEF0 & IDEF1x      [Author's Course]

What you will learn:

• Advanced concepts of both the IDEF0 and IDEF1x methodologies.

# Days: Four (4)

# Students:  Maximum of 10

Cost (minus expenses): 	$800/Person if taken when offered to the public by Wizdom.

$8450  to on-site client firm.

Course Location:  Held publicly at various sites around the world, as well as on-site at a client firm.

COURSE 4 of  4:    IDEF0 & IDEF1x with IDEFine-0 & 1x      [Author's Course]

What you will learn:

•  Advanced concepts of  both the IDEF0 and IDEF1x methodologies using Wizdom Systems' tools.

# Days: Five(5)

# Students:  Maximum of 10

Cost (minus expenses): 	$17,360 w/ one copy of the IDEFine Family tool suite installed/delivered.

$37,360 w/ four copies of the IDEFine Family tool suite installed/delivered.

Course Location:     This course is offered only to on-site client firm only.

THE FOLLOWING EIGHT LISTINGS ARE GROUPED

UNDER WIZDOM SYSTEM'S

IDEFSource Series  Courses

IDEFSource Course 1 of 8   Introduction to IDEF        [Intro Course]

Abstract:

This course is designed for the potential user of IDEF methodologies.  Computers are not used in this introductory course.

What you will learn:

•  Systems engineering methodologies in perspective

•  SADT and IDEF principles

•  IDEF applications in manufacturing, software engineering, and business environments

•  Students will construct IDEF0 and IDEF1 diagrams

# Days: Two (2)

# Students:  Maximum of 15

Cost (minus expenses):	$550 per person (includes instruction, materials, luncheon)

$4550 as an on-site course.

Course Location:   Held publicly at various sites around the world, as well as on-site at a client firm.

IDEFSource Course 2 of 8:   IDEF modeling of manuf. systems. [Reader's/User's Course]

Abstract:

This course shows how to use IDEF0, IDEF1X, and IDEFcost to plan and implement improvements in manufacturing systems.

What you will learn:

•  Project definition and management

•  Data collection

•  Construction of IDEF0 and IDEF1X models

•  Migration from IDEF models to process flow diagrams

•  Specification of TO-BE systems architecture with IDEF.

# Days: Two (2)

# Students:  maximum of 15.

Cost (minus expenses): 	$595 per person (includes instruction, materials, luncheon)

$4550 as an on-site course.

Course Location: Held publicly at various sites around the world, as well as on-site at a client firm.

IDEFSource Course 3 of 8 Enterprise modeling with IDEF  [Reader's/User's Course]

Abstract:

The course shows how IDEF0, IDEF1X, and IDEFcost analysis are used to improve enterprise operations by modeling organizations, their functions, information flows, and interfaces.  This is a course for people who want to re-design their organization's business processes, plan information integration, and control change in a turbulent environment.

What you will learn:

•  How IDEF modeling fits in with CIM, CASE, simulation, and other information management

strategies.

•  How to review basic constructs of IDEF0 and IDEF1X

•  Using a methodology  for using IDEF  as the basis for Activity-Based Costing.

•  To describe IDEFast and IDEFlow - Wizdom's methodology for IDEF based timeline and process

flow analysis.

•  How to present a case study of DOD business system integration planning using IDEF.

# Days: Two (2)

# Students:  maximum of 15.

Cost (minus expenses):  	$595 per person (includes instruction, materials, luncheon)

$4550 as an on-site course.

Course Location:  Held publicly at various sites around the world, as well as on-site at a client firm.

IDEFSource Course 4 of 8:  Software Engineering with IDEF  [Author's Course]

Abstract:

The use of IDEF in CASE tools offers the opportunity for standardization in front-end software engineering.   IDEF is used as a common frame of reference for a systems and database design project.

What you will learn:

•  Basics of IDEF0 and IDEF1X

•  Fundamentals of Business Process Improvement

•  Fundamentals of Data Asset Management

•  Introduction to Corporate Information Management

•  Creating + refining IDEF1X models,

•  Developing TO-BE models using IDEF

•  Collecting Data and Managing IDEF Projects

•  SW Engineering Basics (system life cycle, issues in analysis, design, prototyping, coding, testing,

installation and maintenance.  Standards for SW engineering documentation. Role of CASE tools.

•  Using IDEF in Software Engineering

# Days: Two (2)

# Students:  maximum of 15.

Cost (minus expenses):  	$595 per person (includes instruction, materials, luncheon)

$4550 as an on-site course.

Course Location: Held publicly at various sites around the world, as well as on-site at a client firm.

IDEFSource Course 5 of 8 TQM with IDEF     [Reader's/User's Course]

Abstract:

Total Quality management is increasingly a requirement in major DOD and civilian  contracts.  This course uses IDEF methods to describe the basic re-orientation required for TQM; and to describe the implementation of TQM in operation, marketing, purchasing, human resources, and corporate culture.

# Days: Two (2)

# Students:  maximum of 15.

Cost (minus expenses): 	$595 per person (includes instruction, materials, luncheon)

$4550 as an on-site course.

Course Location: Held publicly at various sites around the world, as well as on-site at a client firm.

IDEFSource Course 6 of 8 :Business Process Re-Design w/ IDEF  [Reader's/User's Course]

Abstract:

IDEF is used to plan business improvements prior to implementing new policies, procedures, and computer systems.  In DOD's Corporate Information Management program, IDEF modeling is the first step in developing new systems.  Planners model the way business is done today and use the model to plan improvement, often through a non-computer solution to information problems such as work simplification.

What you will learn:

•  How IDEF modeling fits in with CIM, CASE, simulation, and other information management

strategies.

•  How to review basic constructs of IDEF0 and IDEF1X

•  A methodology  for using IDEF  as the basis for Activity-Based Costing.

•  How to present a case study of DOD business system integration planning, using IDEF.

# Days: Two (2)

# Students:  maximum of 15.

Cost (minus expenses):	$595 per person (includes instruction, materials, luncheon)

$4550 as an on-site course.

Course Location: Held publicly at various sites around the world, as well as on-site at a client firm.

IDEFSource Course 7 of 8:  Process Flow Modeling w/ IDEF   [Reader's/User's Course]

What you will learn:

•  Concepts for modeling process flow using the IDEF methodology.

# Days: Two (2)

# Students:  maximum of 15.

Cost (minus expenses): 	 $595 per person (includes instruction, materials, luncheon)

$4550 as an on-site course.

Course Location: Held publicly at various sites around the world, as well as on-site at a client firm.

IDEFSource Course 8 of 8  Activity Based Costing w/ IDEF   [Reader's/User's Course]

What you will learn:

•  Concepts of activity based costing using the IDEF methodology.

# Days: Two (2)

# Students:  maximum of 15.

Cost (minus expenses): 	$595 per person (includes instruction, materials, luncheon)

$4550 as an on-site course.

Course Location: Held publicly at various sites around the world, as well as on-site at a client firm.

NOTES:

Tools/Platforms Used:

These courses are taught using Wizdom System's own IDEFine Family _ tool suite.

Materials Used/Kept by Students:

A book of slides  is used and kept by the students.

IDEF1/IDEF1X Training Offered?:

Many of the courses include IDEF1X training.  IDEF1 is not addressed at all.

Many Exercises/Hands On?:

Yes. Each class  has a lot of hands-on exercises to be worked through.

Can Course be Tailored to SW Process Modeling?:

The software engineering with IDEF course  has a lot of useful information, though it doesn't seem specifically aimed at software process modeling.

Misc:

•  Wizdom claims that "virtually any program can be structured to suit the needs of the particular client firm."

• The customer must provide all computers to be used, for all classes, except the Introduction to IDEF course,

in which no computers are used.